Table of Contents

Chapter 1 IntroductioN	2

1.1 Features	2

1.2 Outline	3

Chapter 2 General specification	4

2.1 Tube Display	4

2.2 Electricity	5

2.3 Overall dimensions	6

2.4 Environment	6

2.5 Driver interface	6

2.6 DIP switch	7

2.6.1 DSP-800F dip switch setting 	7

2.6.2 DSP-900 dip switch setting	9

Chapter 3 Interface	10

3.1 Interface	10

3.2 Interface conversion adapter	12

Chapter 4 Command Description	15

4.1 DSP-800F command set	15

4.1.1 DSP-800F command mode	15

4.1.2 EPSON command mode	17

4.1.3 UTC/S command mode	18

4.1.4 UTC/P command mode	18

4.1.5 AEDEX command mode	19

4.1.6 ULTIMATE command mode	19

4.1.7 DSP800 command mode	21

4.1.8 CD5220 command mode	22

4.2 DSP-900 command set	24

Chapter 5 character set	26

5.1 Control code set	26

5.2 U.S.A font set	26

5.3 International character selection	27

Chapter 6 INSTALLATION GUIDE	28

# Chapter 1 Introduction

## 1.1	Features

	The DSP-800F is a Vacuum Fluorescent Display which displays 20 columns and 2 lines, and the DSP-900 is a Vacuum Fluorescent Graphic Display which displays 192x16 dots.

	Blue-green fluorescent color is easy on the eyes.

	The display panel is movable so that it can be adjusted for the best viewing angle.

	The customer display have different height by adjusting the support.

	The interface of customer display is RS-232C, with baud rates from 1200 up to 9600 bps.

	The customer display have provided the pass through function to reduce the cable connection.

	The selftest function check the circuit board and performs each function for diagnostic 	purpose.

	The user defined and international character sets are the standard of customer display.

	The DSP-900 has support 512K bytes customer character ROM.

## 1.2 	Outline

The customer display outline have included of three parts: the panel, the support, and the interface adapter

your VFD customer display should has following accessories:

| Item | Part Number | Dimension (mm) | Qty | Description |
| --- | --- | --- | --- | --- |
| 1 | MP221R44VFD01 |  | 1 | Utility Diskette |
| 2 | MP150CD200200 |  | 1 | User Menu |
| 3 | MP01FIN5V1AK0* | 46 (W) x 71 (D) x 40 (H) | 1 | Adapter 5V/1A (UL, TUV,CSA) |
| 4 | MP53TP308DNI0 |  | 4 | Screw P3x8 for Interface conv. unit |
| 5 | MP53WF3R1FCL0 |  | 4 | Screw for F3.1x15 interface conv. unit |
| 6 | SA02CD2002A00 | 224 (W) x 92(D) x 48(H) | 1 | Panel for DSP-800F |
| 7 | MP44VFDSUPTS0 | 88 (H), 25 (dim.) | 1 | Support (short) |
| 8 | MP44VFDSUPTS0 | 220 (H), 25 (dim.) | 1 | Support (long) |
| 9 | MP07BASE00ICD | 182(W)x860(D)x3(H) | 1 | Metal parts |
| 10 | SA01CD2002BV2 | 186(W)x950(D)x50(H) | 1 | Interface conversion adapter |
| 11 | SA02CD9216A00 | 224 (W) x 92(D) x 48(H) | 1 | Panel for DSP-900 |

# Chapter 2 General specification

## 2.1	Tube Display

(i) DSP-800F

| Customer display | Vacuum Fluorescent Display Blue green |
| --- | --- |
| Display pattern | 5 x 7 dot matrix |
| Brightness | 700 cd/m2 |
| Character type | 95 Alphanumeric & 32 international characters |
| Character size | 6.4 mm (W) x 9.2 mm (H) |
| Character number | 20 x 2 |
| Character pitch | Refer the figure 2.1 |

(ii) DSP-900

| Customer display | Vacuum Fluorescent Graphic Display Blue green |
| --- | --- |
| Display pattern | 8x8 or 16x16 dot matrix |
| Display Area (X x Y) | 140.6 x 35.0 mm |
| Number of dots (X x Y) | 192x16 |
| Dot size (X x Y) | 0.4 x 0.4 mm |
| Luminance | 515 cd /m2 |
| Character type | 95 Alphanumeric & 32 international characters (for 8x8 font) |

## 2.2	Electricity

| Power source | DC +5V |  |
| --- | --- | --- |
| Power consumption | 15 watts maximum 3 watts average |  |
| Central control unit | CPU ROM RAM | 8031 BH 32K flash ROM 32K SRAM |
| Speed | CPU | 11.0592 MHz |
| Connector | 8 pins phone jack, D-SUB 9, and 25 pins (female) connector |  |

## 2.3	Overall dimensions

| Dimension (panel) | 225 mm (W) x 50 mm (D) x 92 mm (H) |
| --- | --- |
| Dimension (support) | (382/298/160/74) mm (H) x 33 dia. mm |
| Dimension (base) | 190 mm (W) x 96 mm (D) x 27 mm (H) |
| Viewing angle | Max. 45 |
| Horizontal rotation | Max. 340 |
| Weight | About 0.8 Kg |

## 2.4	Environment

| Operating temperature | +10℃ to +40℃ |
| --- | --- |
| Storage temperature | -10℃to +50℃ |
| Relative humidity | 0% to 90% RH |

## 2.5	Driver interface

| Driver interface | RS232 |
| --- | --- |

## 2.6	DIP switch settings

The default protocol is 9600 bps, non-parity, 8 data bits, 1 stop bit and with DTR/DSR control.

## 2.6.1 DSP-800F dip switch setting (Version 3.04)

(i) baud Rate select

| SW number SW1 | Function description baud rate(bps) |
| --- | --- |
| OFF | 9600 |
| ON | 4800 |

(ii) command type select

| SW number |  |  | Function description | Software Defined |
| --- | --- | --- | --- | --- |
| SW4 | SW3 | SW2 | Command type | Hex code |
| OFF | OFF | OFF | DSP-800F | 00 |
| OFF | OFF | ON | EPSON Pos D101/D202 | 01 |
| OFF | ON | OFF | UTC/S | 02 |
| OFF | ON | ON | UTC/P | 03 |
| ON | OFF | OFF | AEDEX | 04 |
| ON | OFF | ON | ULTIMATE | 05 |
| ON | ON | OFF | DSP800 | 06 |
| ON | ON | ON | CD5220 | 07 |

(iii) international character set

| SW number |  |  |  | Function description |  |
| --- | --- | --- | --- | --- | --- |
| SW8 | SW7 | SW6 | SW5 | International character set(code 20H-7FH) | Code table (code 80H-FFH) |
| OFF | OFF | OFF | OFF | U.S.A. | PC-437 (USA, standard European) |
| OFF | OFF | OFF | ON | FRANCE | PC-850(multilingual) |
| OFF | OFF | ON | OFF | GERMANY | PC-850(multilingual) |
| OFF | OFF | ON | ON | U.K. | PC-850(multilingual) |
| OFF | ON | OFF | OFF | DENMARK I | PC-850(multilingual) |
| OFF | ON | OFF | ON | SWEDEN | PC-850(multilingual) |
| OFF | ON | ON | OFF | ITALY | PC-850(multilingual) |
| OFF | ON | ON | ON | SPAIN | PC-850(multilingual) |
| ON | OFF | OFF | OFF | JAPAN | Katakana |
| ON | OFF | OFF | ON | NORWAY | PC-850(multilingual) |
| ON | OFF | ON | OFF | DENMARK II | PC-850(multilingual) |
| ON | OFF | ON | ON | SLAVONIC/RUSSIAN |  |
| ON | ON | OFF | OFF | Factory define |  |
| ON | ON | OFF | ON | Factory define |  |
| ON | ON | ON | OFF | Factory define |  |
| ON | ON | ON | ON | Factory define |  |

## 2.6.2 DSP-900 dip switch setting

(i) baud Rate setting:

| SW8 | SW7 | Baud rate(bps) |
| --- | --- | --- |
| OFF* | OFF* | 9600 |
| OFF | ON | 4800 |
| ON | OFF | 2400 |
| ON | ON | 1200 |

(ii) power on string function enable/disable:

| SW2 | Function description |
| --- | --- |
| OFF* | Enable |
| ON | Disable |

(iii) selftest function:

| SW1 | Function description |
| --- | --- |
| OFF* | Disable |
| ON | Enable |

# Chapter 3 Interface

## 3.1 Interface

-Specifications

Data transmission method	: Asynchronous Serial.

Handshaking 				: DTR/DSR control

Default protocol			: 9600 bps, non-parity, 8 data bits, 1 stop bit.

-Interface connector (display panel side)

-RJ45 type 8 pin modular jack

-Pin assignment:

-Connector signal assignment

| Pin no. |  | I/O | Description |
| --- | --- | --- | --- |
| 1 | NC |  | No connection |
| 2 | TXD | OUTPUT | Transmit data |
| 3 | RXD | INPUT | Receive data |
| 4 | DSR | INPUT | Data set ready (from the printer) |
| 5 | GROUND |  |  |
| 6 | DTR | OUTPUT | Data terminal ready (from the display) |
| 7 | Vp |  | Power supply (+5V) |
| 8 | Vgnd |  | Power supply (GND) |

-Communication Protocol

1. 	Receive data.

The DTR signal is as follows:

[HIGH] 	This indicates that the display isn't ready to receive data.

It depend on the following conditions:

 The period from when the power is turned on to when the printer first becomes ready to receive data.

	When the remaining space in the receive buffer becomes 128 bytes or less.

 When the DTR signal of the printer is HIGH when the printer is selected using the command.

[LOW] 	This indicates that the display is ready to receive data.

It depend on the following conditions:

 When the printer first becomes ready to receive data after power-on.

	When the remaining space in the receive buffer becomes 128 bytes or more.

 When the DTR signal of the printer is LOW when the printer is selected using the command.

2. 	Transmit data.

After confirming the DSR is LOW, data is transmitted to printer.

## 3.2 Interface conversion adapter

The interface adapter section has connectors for the display panel, the printer, the power supply, and host computer. All the data transmitted from the host computer will be received by the display. If this data is for the display, the data will be processed, and if it is for the printer, it will be transmitted to the printer. Whether the data is for the display or the printer can be switched using the peripheral device selection command.

D-sub 25 pin connector signal assignment

| Pin no. |  | I/O | Description |
| --- | --- | --- | --- |
| 1 | N.C. |  | No Connection |
| 2 | TXD | OUTPUT | Transmit Data |
| 3 | RXD | INPUT | Receive Data from DB-9 Pin#2 |
| 4 | By Selection |  | N.C. or +5V |
| 5 | GND |  | Power GND |
| 6 | DSR | INPUT | Data Set Ready |
| 7 | GND |  | Power GND |
| 20 | DTR | OUTPUT | Data Terminal Ready from DB-9 Pin#4 |
| 22 | By Selection |  | N.C. or +5V |

D-sub 9 pin connector signal assignment

| Pin no. |  | I/O | Description |
| --- | --- | --- | --- |
| 1 | NC |  | No Connection |
| 2 | By Pass |  | Connect with DB-25 Pin#3 |
| 3 | RXD | INPUT | Receive Data |
| 4 | By Pass |  | Connect with DB-25 Pin#20 |
| 5 | GND |  | Power GND |
| 6 | DTR | OUTPUT | Data Terminal Ready |
| 7 | By Selection |  | N.C. or +5V |
| 8 | GND |  | Power GND |
| 9 | By Selection |  | N.C. or +5V |

D-sub 9 pin & 25 pine Power Source signal assignment

| Jumper number | Description |
| --- | --- |
| 1-2 | The +5V power source is assign to No. 9 of 9 pin D-sub |
| 3-4 | The +5V power source is assign to No. 7 of 9 pin D-sub |
| 5-6 | The +5V power source is assign to No. 22 of 25 pin D-sub |
| 7-8 | The +5V power source is assign to No. 4 of 25 pin D-sub |

The factory setting for this jumper is NULL, that means the customer display must have the +5V DC adapter. If there has any reason that user doesn't want to use the adapter, the engineer should set this jumper and provide the +5V power source to the correct pin number.

# Chapter 4 Command Description

## 4.1 DSP-800F command set (Version 3.04)

### 4.1.1 DSP-800F command mode

| Command | Code description(hex) | Function description |
| --- | --- | --- |
| HT | 09 | Move cursor right(only valid in overwrite mode) |
| BS | 08 | Move cursor left(only valid in overwrite mode) |
| CR | 0D | Move cursor to left-most position(only valid in overwrite mode) |
| ESC @ | 1B 40 | Initialize customer display to initial state, clears display buffer, set display mode to shift and sets current display row to upper row |
| ESC U | 1B 55 | Select upper row as current row(initial default) |
| ESC D | 1B 44 | Select lower row as current row |
| ESC Aψ | 1B 41ψ | Sets customer display disable or enable ψ- ’D’=disable,’E’=enable |
| ESC C r c | 1B 43 r c | Move cursor to specified position(only valid in overwrite mode) -r Row( ‘U’=upper, ’D’=lower -c Column number(range from 1~20) |
| ESC E rψ | 1B 45 rψ | Set special effect or display mode of specified row |
| ESC R n | 1B 52 n | Set international font sets -n international fonts code |
| ESC = n | 1B 3D n n=’1’-’3’ | Select peripheral -n ‘1’=printer only, ‘2’=customer display only, 3=both peripheral |
| ESC % n | 1B 25 n | Set font pattern -n 0=normal font set, 1=user font set |
| ESC & n s [p] | 1b 46 n s data | Define user font pattern -n code for first character -s code for last character -data 5 bytes required for each character |

(REMARK)*Using commands “ESC E rψ”, the value of parameter

r	58=all rows

55=upper row

44=lower row

ψ special function, the value is one of

30= shift mode(default display mode)

31=rotation mode

32=blink mode

33=clear this row and switch to shift mode

34=overwrite mode

35=vertical mode

*Set international font set

| n | International font set |
| --- | --- |
| 30h 31h 32h 33h | U. S.A GERMANY FRANCE JAPAN |

### 4.1.2 EPSON command mode

| Command | Code description(hex) | Function description |
| --- | --- | --- |
| HT | 09 | Move cursor right |
| BS | 08 | Move cursor left |
| US LF | 1F 0A | Move cursor up |
| LF | 0A | Move cursor down |
| US CR | 1F 0D | Move cursor to right-most position |
| CR | 0D | Move cursor to left-most position |
| HOM | 0B | Move cursor to home position |
| US B | 1F 42 | Move cursor to bottom position |
| US $ x y | 1F 24 x y x=1-20 y=01,02 | Move cursor to specified position |
| CLR | 0C | Clear display screen |
| CAN | 18 | Clear cursor line |
| US X n | 1F 58 n 01<=n<=04 | Brightness adjustment |
| US E n | 1F 45 n n=00-ff | Blink display screen |
| ESC @ | 1B 40 | Initialize display |
| ESC t n | 1B 74 n n=00-0f | Select character code table |
| ESC R n | 1B 52 n n=00-0f | Select international character set |
| US r n | 1F 72 n n=00,01 | Select/cancel reverse character |
| US MD1 | 1F 01 | Specify overwrite mode |
| US MD2 | 1F 02 | Specify vertical scroll mode |
| US MD3 | 1F 03 | Specify horizontal scroll mode |
| ESC & s n m [a(pl..pa)x m-n | 1B 26 1 n m [a(pl..pa)]x m-n 20<n<=m<=ff | Define download characters 20<n<=m<=ff a=1-5 p1..p5=row1..row5 |
| ESC ? n | 1B 3F n n= 20-7e | Cancel user-defined characters |
| ESC % n | 1B 25 n n=00,01 | Select/cancel download character set |
| ESC W n s x1y1x2y2 | 1b 57 n s x1 y1 x2 y2 n=1,2,3,4 s=0,1 | Specify/cancel the window range 1<=x1<=x2<=20 1<=y1<=y2<=2 |
| ESC = n | 1B 3D n n=1,31 ,select printer n=2,32 ,select display | Select peripheral device |
| US : | 1F 3A | Set starting/ending position of macro definition |
| US ^ n m | 1F 5E n m 00<=(n,m)<=ff | Execute and quit macro |
| US @ | 1f 40 | Execute self-test |
| US T h m | 1f 54 h m 0<=h<=17 , 0<=m<=3b | Dispaly time |
| US U | 1F 55 | Display time continuously |

### 4.1.3 UTC/S command mode(UTC standard mode)

| Command | Code description(hex) | Function description |
| --- | --- | --- |
| BS | 08 | Back space |
| HT | 09 | Horizontal tab |
| LF | 0A | Line feed |
| CR | 0D | Carriage return |
| DLE | 0F | Display position |
| DC1 | 11 | Over write display mode |
| DC2 | 12 | Vertical scroll mode |
| DC3 | 13 | Cursor on |
| DC4 | 14 | Cursor off |
| ESC d | 1B 64 | Change to UTC enhanced mode |
| US | 1F | Clear display |

### 4.1.4 UTC/P command mode(UTC enhanced mode)

| Command | Code description(hex) | Function description |
| --- | --- | --- |
| ESC u A_.CR | 1B 75 41 [ data x 40]0D | Upper line display |
| ESC u B_.CR | 1B 75 42 [ data x 40]0D | Bottom line display |
| ESC u D_.CR | 1B 75 44 [ data x 40]0D | Upper line message scroll continuously |
| ESC u E_.CR | 1B 75 45 hh ‘:’ mm o 0D h,m=’0’-‘9’ | Display time |
| ESC u F_.CR | 1B 75 46 [ data x 40]0D | Upper line message scroll once pass |
| ESC u H_.CR | 1B 75 48 n m 0D 20h<=n,m | Change attention code |
| ESC u 1_.CR | 1B 75 49 [ data x 40]0D | Two line display |
| ESC RS_.CR | 1B 0F 0D | Change to UTC standard mode |

### 4.1.5 AEDEX command mode

| Command | Code description(hex) | Function description |
| --- | --- | --- |
| ! # 1_.CR | 21 23 31 [ data x 40]0D | Upper line display |
| ! # 2_.CR | 21 23 32 [ data x 40]0D | Bottom line display |
| ! # 4_.CR | 21 23 34 [ data x 40]0D | Upper line message scroll continuously |
| ! # 5_.CR | 21 23 35 hh ‘:’ mm o 0D h,m=’0’-‘9’ | Display time |
| ! # 6_.CR | 21 23 36 [ data x 40]0D | Upper line message scroll once pass |
| ! # 8_.CR | 1B 75 48 n m 0D 20h<=n,m | Change attention code |
| ! # 9_.CR | 21 23 39 [ data x 40]0D | Two line display |

### 4.1.6 ULTIMATE command mode

(1) Line Feeding

| Command | Code description(hex) | Function description |
| --- | --- | --- |
| Cntrl | 0A | DC 1 Mode The cursor moves up or down to another row staying on the same line. DC 2 Mode When the cursor is in the second row the character displayed there, is shifted up to the first row, leaving the cursor at its present position, then the second row is cleared. When the cursor is in the first row, the cursor moves down to the second row. |
| Cntrl M | 0D | The cursor moves to the least significant position of the same row. |
| Cntrl P | 10 | Instead of writing a character in the current position, the write-in starting position can be pointed by using this function. After writing 10HEX to prepare module for this command, another HEX byte is written to specify the position desired. A third byte reprexenting data is then sent. |

ROW CHARACTER POSITION CHART (HEX)

| 1 | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 | 08 | 09 | 0A | 0B | 0C | 0D | 0E | 0F | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 14 | 15 | 16 | 17 | 18 | 19 | 1A | 1B | 1C | 1D | 1E | 1F | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 |

(2) Normal Display Mode (DC 1)

| Command | Code description(hex) | Function description |
| --- | --- | --- |
| Cntrl Q | 11 | After writing a character, the cursor is shifted to the right one position automatically. When the cursor is on the most significant position of the first row. The cursor moves to the least significant position of the second row, when the cursor is on the most significant position of the second row, the cursor moves to the least significant position of the first row. |

(3) Vertical Scroll Mode (DC 2)

| Command | Code description(hex) | Function description |
| --- | --- | --- |
| Cntrl R | 12 | After writing the characters up to the most significant position of the second row, all characters displayed in the second row are shifted to the upper row (first row), clearing the second row. When the power is turned on, this DC 2 Mode is selected, and will be held until another mode is selected. |
| Cntrl -- | 1F | Resetting the module. All characters displayed are erased, then the write-in position (cursor position) is set on the least. |

### 4.1.7 DSP800 command mode

| Command | Code description(hex) | Function description |
| --- | --- | --- |
| EOT SOH I n ETB | 04 01 49 n 17 | Select international character set. |
| EOT SOH P n ETB | 04 01 50 n 17 n=31-58 | Move cursor to specified position |
| EOT SOH C n m ETB | 04 01 43 n m 17 31<=n<=m<=58 | Clear display range from n position to m position and move cursor to n position. |
| EOT SOH S n ETB | 04 01 53 n 17 n=31-33 | Save the current displaying data to n layer for demo display |
| EOT SOH D n m ETB | 04 01 44 n 17 n=31-37 m=31-33 | Display the saved data |
| EOT SOH A n ETB | 04 01 41 n 17 n=31-34 | Brightness adjustment |
| EOT SOH F n ETB | 04 01 46 n 17 00<=n<=FF | Blink display screen |
| EOT SOH = n ETB | 04 01 3D n 17 n=’1’,’2’ | Select peripheral device N=’1,’, printer n=’2’, display |
| EOT SOH % ETB | 04 01 25 17 | Initialize display |
| EOT SOH @ ETB | 04 01 40 17 | Execute self-test |

(REMARK)* About the command display the saved data

| n | lay select | n | lay select | m | show mode |
| --- | --- | --- | --- | --- | --- |
| bit 0=1 | lay 1 | bit 3=1 | lay 4 | bit 0=1 | show mode 1 |
| bit 1=1 | lay 2 | bit 4=0 | lay 5 | bit 1=1 | show mode 2 |
| bit 2=1 | lay 3 |  |  |  |  |

### 4.1.8 CD5220 command mode

| Command | Code description(hex) | Function description |
| --- | --- | --- |
| ESC DC1 | 1B 11 | Overwrite mode |
| ESC DC2 | 1B 12 | Vertical scroll mode |
| ESC DC3 | 1B 13 | Horizontal scroll mode |
| ESC Q A CR | 1B 51 41 [N]x20 0D | Set the string display mode, write string to upper line |
| ESC Q B CR | 1B 51 42 [N]x20 0D | Set the string display mode, write string to lower line |
| ESC Q D CR | 1B 51 44 [N]xm20 0D m<40 | Upper line message scroll continuously |
| ESC [ D | 1B 5B 44 | Move cursor left |
| BS | 08 | Move cursor left |
| ESC [ C | 1B 5B 43 | Move cursor right |
| HT | 09 | Move cursor right |
| ESC [ A | 1B 5B 41 | Move cursor up |
| ESC [ B | 1B 5B 42 | Move cursor down |
| LF | 0A | Move cursor down |
| ESD [ H | 1B 5B 48 | Move cursor to home position |
| HOM | 0B | Move cursor to home position |
| ESC [ L | 1B 5B 4C | Move cursor to left-most position |
| CR | 0D | Move cursor to left-most position |
| ESC [ R | 1B 5B 52 | Move cursor to right-most position |
| ESC [ K | 1B 5B 4B | Move cursor to bottom position |
| ESC l x y | 1B 6C x y 1<=x<=20, y=1,2 | Move cursor to specified position |
| ESC @ | 1B 40 | Initialize display |
| ESC W s x1 x2 y | 1B 57 1 x1 x2 y 1<=x1<=x2<=20 y=1,2 | Set or cancel the window range at horizontal scroll mode |
| CLR | 0C | clear display screen, and clear string mode |
| CAN | 18 | clear cursor line, and clear string mode |
| ESC * n | 1B 2A n 1<=n<=4 | brightness adjustment |
| ESC & s n m [a(pl..pa)]x (m-n+1) | 1b 26 1 n m [a(pl..pa)]x (m-n+1) 20<=n<=m<=7F | define download characters a=1-5 p1..p5=row1..row5 |

| ESC ? n | 1B 3F n 20<=n<=7F | delete download characters |
| --- | --- | --- |
| ESC % n | 1B 25 n n=1,0 | select/cancel download character set. |
| ESC _ n | 1B 5F n n=1,0 | set cursor ON/OFF |
| ESC f n | 1B 66 n | select international fonts set |
| ESC c n | 1B 63 n | select fonts, ASCII code or JIS code |
| ESC = n | 1B 3D n n=1,2,3,31,32,33 | select peripheral device, Display or Printer |

(REMARK)

*While using command “ESC QA” or “ESC QB”, these two commands could using combine with terminal printer—TP 2688 or TP3688

* If using commands “ESC QA” or “ESC QB”, others commands cannot be used except using command “CLR” or “CAN” to change operating mode.

* If using commands “ESC QD”, upper line message move continuously, till receive a new command, and clear upper line and move cursor to upper line left-most position.

Set international font set							Select code Table

| n | International font set | n | International font set |  | n | International |
| --- | --- | --- | --- | --- | --- | --- |
| A G I J U F S | U. S.A GERMANY ITALY JAPAN U. K. FRANCE SPAIN | N W D E L R | NORWAY SWEDEN DENMARK I DENMARK II SLAVONIC RUSSIA Reserved |  | A J R L | compliance with ASCII code compliance with JIS cocde compliance with RUSSIA code compliance with SLAVONIC code |

## 4.2 DSP-900 command set

| Command | Code description(hex) | Function description |
| --- | --- | --- |
| HT | 09 | Move cursor right(only valid in overwrite mode) |
| BS | 08 | Move cursor left(only valid in overwrite mode) |
| CR | 0D | Move cursor to left-most position(only valid in overwrite mode) |
| ESC @ | 1B 40 | Initialize customer display to initial state, clears display buffer, set display mode to shift and sets current display row to upper row |
| ESC U | 1B 55 | Select upper row as current row(initial default) |
| ESC D | 1B 44 | Select lower row as current row |
| ESC Aψ | 1B 41ψ | Sets customer display disable or enable ψ- ’D’=disable, ’E’=enable |
| ESC C r c | 1B 43 r c | Move cursor to specified position(only valid in overwrite mode) -r Row( ‘U’=upper,’ D’=lower -c Column number(range from 1~20) |
| ESC E rψ | 1B 45 rψ | Set special effect or display mode of specified row |
| ESC R n | 1B 52 n | Set international font sets -n international fonts code |
| ESC = n | 1B 3D n n=’1’-’3’ | Select peripheral -n ‘1’=printer only, ‘2’=customer display only ‘3’=both peripheral |
| ESC % n | 1B 25 n | Set font pattern -n 0=normal font set 1=user font set 2=chinese character font |
| ESC & n s [p] | 1b 46 n s data | Define user font pattern -n code for first character -s code for last character -data 5 bytes required for each character |
| ESC * x y w h [b] | 1b 2A x y w h [b] | -x Initial upper left x position(0 to 191) -y Initial upper left x position(0/1) -w bit image width (1 to 192) -h bit image height(1/2) -[b] bit image string |

(REMARK)

*Display Chinese font

step 1: send “ESC % 2” to let DSP-900 enter Chinese mode

step 2: send any ‘BIG 5’ code to DSP-900 for Chinese character display

*Using commands “ESC E rψ”, the value of parameter

r	58=all rows

55=upper row

44=lower row

-ψ  special function, the value is one of

30=shift mode(default display mode)

31=rotation mode

32=blink mode

33=clear this row and switch to shift mode

34=overwrite mode

35=vertical mode

*Set international font set

| n | International font set |
| --- | --- |
| 30h 31h 32h 33h | W. S.A GERMANY FRANCE JAPAN |

Chapter 5 Character Set

## (1)Control code set

| HEX | CODE | HEX | CODE |
| --- | --- | --- | --- |
| 00H | NULL | 10H | DLE |
| 01H | MD1 | 11H | DC1 |
| 02H | MD2 | 12H | DC2 |
| 03H | MD3 | 13H | DC3 |
| 04H | MD4 | 14H | DC4 |
| 05H | MD5 | 15H |  |
| 06H | MD6 | 16H |  |
| 07H | MD7 | 17H |  |
| 08H | BS,MD8 | 18H | CAN |
| 09H | HT | 19H |  |
| 0AH | LF | 1AH |  |
| 0BH | HOM | 1BH | ESC |
| 0CH | CLR | 1CH |  |
| 0DH | CR | 1DH |  |
| 0EH | SLE1 | 1EH | SF1 |
| 0FH | RS,SLE2 | 1EH | US,SF2 |

## (2)U.S.A font set

(3)International character selection( Indicated character selection by dip switch or command )

CHAPTER 6 INSTALLATION GUIDE