# INTERFACE WITH EXTERNAL DEVICE

## INTERFACE with RS-232C

    - 7-BIT ASCII code

    - Even parity

    - 1 stop bit

    - 9600 baud rate

### TYPE-2 INTERFACE

:  Discontinual RS-232C Interface

    - SHARP ER-AXXX, ER-A450T, New SANYO ECRs using RS-232,TOLEDO 3213 etc.

#### 1) PROTOCOL

ECR				SCALE(PD-II)

Command  ------------

<W>

------------	Response

<STX> 0XXXX <CR>   :  lb weighing mode

or  <STX> XXXXX <CR>   :  kg weighing mode

Error message   :  <STX>?<status byte><CR>

== STATUS BYTE ==

| PARITY BIT | ALWAYS==1 |  | ZERO |  | MINUS | OVERLOAD | MOTION |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit3 | Bit 2 | Bit 1 | Bit 0 |

cf)	W : 57H (ASCII code)

STX : 02H (ASCII code)

CR  : ODH (ASCII code)

Ex)

Weight  :  12.34 lb

ECR			SCALE

W<57H>   ------

------  	<02H><30H><31H><32H><33H><34H><0DH>  : ASCII code

STX   0    1     2    3     4    CR

### TYPE-3 INTERFACE

:  Continual RS-232C Interface

    - SHARP ER-AXXX, New SANYO ECRs using RS-232,TOLEDO 3213 etc

#### 1) PROTOCOL

ECR				SCALE(PD-II)

Command  ------------

<W>

------------	Response

<STX> 0XXXX <CR>   :  lb weighing mode

<STX> XXXXX <CR>   :  kg weighing mode

Error message   :  <STX>?<status byte><CR>

<CR>  ------------	Stop transmitting data

Ex)

Weight  :  12.34 lb

ECR			SCALE

W<57H>   ------

------	<02H><30H><31H><32H><33H><34H><0DH>  : ASCII code

STX   0    1     2    3     4    CR

<02H><30H><30H><30H><30H><30H><0DH>  : ASCII code

STX   0    0     0    0     0    CR

------	<02H><3FH><44H><0DH>

STX   ?  MINUS   CR

#### 2) TRANSMISSION PROCEDURE

(1) PD-II sends data to External Device whenever weight is changed after receiving <W> signal from the External Device.

  - PD-II stops sending data when receives <CR> signal from the External Device.

External Device

<W>    ------

-----	DATA  (If weight is changed)

------	DATA  (If weight is changed)

<CR>   ------     	Stop transmitting data

### TYPE-0 and TYPE-1 INTERFACE

    - Most P.O.S Systems, ECRs and some TEC P.O.S Systems.

#### 1) PROTOCOL

EXTERNAL DEVICE			SCALE(PD-II)

<ENQ>	------------	Initiate communication

<DC2>	------------	Request of weight data

------------	<ACK>		: Acknowledge the request of weight data

Inquiry

------------	<STX>		: Start Transmission

------------	<ID>		: Scale type identifier

------------	<W5>		: Weight data

------------	<W4>		: Weight data

------------	<W3>		: Weight data

------------	<W2>		: Weight data

------------	<W1>		: Weight data

------------	<BCC>		: Block Check Character

------------	<ETX>		: End Transmission

  - Scale Type Identifier

| 41H = 15 kg | 44H = 30 lb |
| --- | --- |
| 43H = 6 kg | 46H = 15 lb |
| NA = 3 kg | NA = 6 lb |
| 42H = 25 kg | 45H = 50 lb |

  - Block Check Character

: <BCC> has all data bytes except <STX> and <ETX> through exclusive OR(XOR).

* Parity Bit : Even

- Data Byte :  <STX><ID><W5><W4><W3><W2><W1><BCC><ETX>

### TYPE-4 INTERFACE

    - NCI ECR(NCR2170), SAMSUNG ER-5100,ER-5115, CRS .etc

#### 1) PROTOCOL

<W> 		------------

<CR>		------------

Inquiry

------------	<LF> XX.XXX  LB <CR>

------------	<LF> S b1b2 <CR><ETX>

lb CASE

------------	<LF> XX.XXX  KG <CR>

------------	<LF> S b1b2 <CR><ETX>

      - XX.XXX  =  Weight value

      - LB  =  The Characters L and B

      - KG  =  The Characters K and G

      - S    =  The Character S

      - b1b2 =  Two status Characters

        - Status Bytes

| Bit7 | Parity Bit | Parity Bit |
| --- | --- | --- |
| Bit6 | 0 | 0 |
| Bit5 | 1 (Always 1) | 1 (Always 1) |
| Bit4 | 1 (Always 1) | 1 (Always 1) |
| Bit3 | 0 | 0 |
| Bit2 | 0 | 0 |
| Bit1 | 1 = Scale at Zero 0 = Not at Zero | 1 = Over Capacity 0 = Not Over Capacity |
| Bit0 | 1 = Scale in Motion 0 = Stable | 1 = Under Capacity 0 = Not Under Capacity |

        - Simplified Status Codes

| B1 | B2 | STATUS Definition |
| --- | --- | --- |
| ASCII Character (ASCII Code) | ASCII Character (ASCII Code) |  |
| 0 (30H) | 0 (30H) | OK |
| 1 (31H) | 0 (30H) | Motion |
| 2 (32H) | 0 (30H) | Scale at Zero |
| 0 (30H) | 1 (31H) | Under capacity |
| 0 (30H) | 2 (32H) | Over capacity |

### TYPE-5 INTERFACE

    - NCI GENERAL , SAMSUNG ER-5115, ER-5100 and Most P.O.S Software

#### 1) PROTOCOL

<W> 		------------

<CR>		------------

Inquiry

------------	<LF> XX.XXX  LB <CR>

------------	<LF> b1b2 <CR><ETX>

lb CASE

------------	<LF> XX.XXX  KG <CR>

------------	<LF> S b1b2 <CR><ETX>

      - XX.XXX  =  Weight value

      - LB  =  The Characters L and B

      - KG  =  The Characters K and G

      - b1b2 =  Two status Characters

        - Status Bytes

| Bit7 | Parity Bit | Parity Bit |
| --- | --- | --- |
| Bit6 | 0 | 0 |
| Bit5 | 1 (Always 1) | 1 (Always 1) |
| Bit4 | 1 (Always 1) | 1 (Always 1) |
| Bit3 | 0 | 0 |
| Bit2 | 0 | 0 |
| Bit1 | 1 = Scale at Zero 0 = Not at Zero | 1 = Over Capacity 0 = Not Over Capacity |
| Bit0 | 1 = Scale in Motion 0 = Stable | 1 = Under Capacity 0 = Not Under Capacity |

        - Simplified Status Codes

| B1 | B2 | STATUS Definition |
| --- | --- | --- |
| ASCII Character (ASCII Code) | ASCII Character (ASCII Code) |  |
| 0 (30H) | 0 (30H) | OK |
| 1 (31H) | 0 (30H) | Motion |
| 2 (32H) | 0 (30H) | Scale at Zero |
| 0 (30H) | 1 (31H) | Under capacity |
| 0 (30H) | 2 (32H) | Over capacity |

### TYPE-6 INTERFACE

    - 8 Data bit

    - None parity

    - 1 stop bit

    - 9600 baud rate

    - SAMSUNG ECR (ER-670)

#### 1> PROTOCOl

EXTERNAL DEVICE			SCALE(PD-II)

<ENQ>	    ------------	Initiate communication

------------	<ACK>		: Acknowledge the request of weight data

<DC1> or <DC2>  ------------	DC1 : For Weight Data

DC2 : For All Data ( PD-II NOT USE)

------------	Send Data Block

#### 1> The Data Trains

1. “DC1”

| SOH | STX | STA | SIGN | W5 | W4 | W3 | W2 | W1 | W0 | UN1 | UN0 | BCC | ETX | EOT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Command |  | DATA BLOCK |  |  |  |  |  |  |  |  |  | Command |  |  |

        - Remark

    - STA  :  A WEIGHING STATUS OF THE SCALE

SCALE IS STABLE ->”S”,  NOT STABLE -> “U”

    - SIGN  :  SIGN OF THE WEIGHT DATA

ZERO AND POSITIVE WEIGHT -> “ “ , NEGATIVE WEIGHT -> “-“

OVER LOAD -> “F”

    - W5 THROUGH W0 –> WEIGHT DATA

BUT ALL “F” WHEN THE SCALE IS PUT ON OVER LOAD.

    - UN1 THROUGH UN0 -> UNIT OF WEIGHT ( Kg OR Lb)

    - BCC  :  BLOCK CHECK CHARACTER

### TYPE-7 INTERFACE

: RS-232C Interface

    - 8 Data bit

    - None parity

    - 1 stop bit

    - 9600 baud rate

    - SAMSUNG ECR (SPAIN)

#### 1) PROTOCOL

ECR				SCALE(PD-II)

Command  ------------

<$>

------------	Response

XXX.XXX <CR>

cf)	$ : 24H (ASCII code)

“.” : 2EH (ASCII code)

CR  : ODH (ASCII code)

Ex)

Weight  :  12.34 kg

ECR			SCALE

$<24H>   ------

------  	<30H><31H><32H><2EH><3H><34H><0DH>  : ASCII code

0    1     2    .    3    4    CR

### TYPE-8 INTERFACE

    - SAMSUNG (PORTUGAL)

#### 1) PROTOCOL

<W> 		------------

<CR>		------------

Inquiry

------------	<LF> XXX.XX  KG <CR>

------------	<LF> S00 <CR><ETX>

lb CASE

------------	<LF> XX.XXX  KG <CR>

------------	<LF> S b1b2 <CR><ETX>

          - XXX.XX  =  Weight value

          - KG  =  The Characters K and G

          - S    =  The Character S

          - 00   =  <30H><30H>