Multi_RFID Library Manual

Version history

| version | Modify record | Modifier | Modified date |
| --- | --- | --- | --- |
| 1.0 | Drafting |  | 2019-08-31 |

catalogue

Dll file name：RFIDPCSC.dll	4

1 General function	4

1.1 Hid_Init	4

1.2 Hid_Close	4

1.3 Get_dll_ver	4

2 Mifare One Card operation	5

2.1 PS_RF_Reset	5

2.2 PS_RF_Set	5

2.3 PS_RF_Search	5

2.4 PS_RF_Write	6

2.5 PS_RF_Read	6

2.6 PS_RF_ReadValue	7

2.7 PS_RF_WriteValue	7

2.8 PS_RF_IncValue	8

2.9 PS_RF_DecValue	8

2.10 PS_RF_BackUpValue	9

2.11 PS_RF_CardSleep	9

3 Error code	9

Dll file name：RFIDPCSC.dll

# 1 General function

## 1.1 Hid_Init

| Function name | Hid_Init |
| --- | --- |
| Function prototype | int32_t WINAPI Hid_Init(); |
| Functional description | Open the RFID device |
| Input parameters | NULL |
| Output parameters | NULL |
| Return value | 0 = operation successfully Other = operation failed |
| prerequisites | NULL |

## 1.2 Hid_Close

| Function name | Hid_Close |
| --- | --- |
| Function prototype | void WINAPI Hid_Close(); |
| Functional description | Close the RFID device |
| Input parameters | NULL |
| Output parameters | NULL |
| Return value | NULL |
| prerequisites | Hid_Init() succeeded |

## 1.3 Get_dll_ver

| Function name | Get_dll_ver |
| --- | --- |
| Function prototype | void WINAPI Get_dll_ver(uint8_t * verData); |
| Functional description | Get DLL version information |
| Input parameters | NULL |
| Output parameters | verData ： DLL version information |
| Return value | NULL |
| prerequisites | NULL |

# 2 Mifare One Card operation

## 2.1 PS_RF_Reset

| Function name | rf_antenna_sta |
| --- | --- |
| Function prototype | int32_t WINAPI PS_RF_Reset(); |
| Functional description | The reader resets and puts the reader into a low power state |
| Input parameters | NULL |
| Output parameters | NULL |
| Return value | = 0 operation successfully < 0 operation failed |
| prerequisites | Hid_Init() succeeded |

## 2.2 PS_RF_Set

| Function name | PS_RF_Set |
| --- | --- |
| Function prototype | int32_t WINAPI PS_RF_Set(int32_t inMode); |
| Functional description | RFID RF switch |
| Input parameters | inMode =0 Turn on the RFID RF switch =1 Turn off the RFID RF switch |
| Output parameters | NULL |
| Return value | = 0 operation successfully < 0 operation failed |
| prerequisites | Hid_Init() succeeded |

## 2.3 PS_RF_Search

| Function name | PS_RF_Search |
| --- | --- |
| Function prototype | int32_t WINAPI PS_RF_Search(uint8_t *cardId, int32_t * cardIdLen); |
| Functional description | Search card |
| Input parameters | NULL |
| Output parameters | cardId RFID card number cardIdLen RFID card number length |
| Return value | = 0 operation successfully < 0 operation failed |
| prerequisites | Hid_Init() succeeded |

## 2.4 PS_RF_Write

| Function name | PS_RF_Write |
| --- | --- |
| Function prototype | int32_t WINAPI PS_RF_Write(int32_t inBlock,int32_t inKeyMod,uint8_t* inKeyData,uint32_t inKeyDataLen,uint8_t* inData,uint32_t inDataLen); |
| Functional description | Writing S50 CARD |
| Input parameters | InBlock Block number 0-255 InKeyMod Key type 0=AKEY 1=BKEY inKeyDataLen Key data length InKeyData Key data InData Write card data inDataLen Write card data length |
| Output parameters | NULL |
| Return value | = 0 operation successfully < 0 operation failed |
| prerequisites | Hid_Init() succeeded |

## 2.5 PS_RF_Read

| Function name | PS_RF_Read |
| --- | --- |
| Function prototype | int32_t WINAPI PS_RF_Read(int32_t inBlock,int32_t inKeyMod,uint8_t* inKeyData,uint32_t inKeyDataLen,uint8_t* inData,uint32_t *inDataLen); |
| Functional description | Reading S50 CARD |
| Input parameters | InBlock Block number 0-255 InKeyMod Key type 0=AKEY 1=BKEY inKeyDataLen Key data length InKeyData Key data |
| Output parameters | inDataLen Read card data inData Read card data length |
| Return value | = 0 operation successfully < 0 operation failed |
| prerequisites | Hid_Init() succeeded |

## 2.6 PS_RF_ReadValue

| Function name | PS_RF_ReadValue |
| --- | --- |
| Function prototype | int32_t WINAPI PS_RF_ReadValue(int32_t inBlock,int32_t inKeyMod,uint8_t* inKeyData,uint32_t inKeyDataLen,long * outDataVal); |
| Functional description | S50 card inquiry balance |
| Input parameters | InBlock Block number 0-255 InKeyMod Key type 0=AKEY 1=BKEY inKeyDataLen Key data length InKeyData Key data |
| Output parameters | inDataVal e-wallet amount |
| Return value | = 0 operation successfully < 0 operation failed |
| prerequisites | Hid_Init() succeeded |

## 2.7 PS_RF_WriteValue

| Function name | PS_RF_WriteValue |
| --- | --- |
| Function prototype | int32_t WINAPI PS_RF_WriteValue(int32_t inBlock,int32_t inKeyMod,uint8_t* inKeyData,uint32_t inKeyDataLen,long inDataVal); |
| Functional description | S50 card initialization wallet |
| Input parameters | InBlock Block number 0-255 InKeyMod Key type 0=AKEY 1=BKEY inKeyDataLen Key data length InKeyData Key data inDataVal e-wallet amount |
| Output parameters | NULL |
| Return value | = 0 operation successfully < 0 operation failed |
| prerequisites | Hid_Init() succeeded |

## 2.8 PS_RF_IncValue

| Function name | PS_RF_IncValue |
| --- | --- |
| Function prototype | int32_t WINAPI PS_RF_IncValue(int32_t inBlock,int32_t inKeyMod,uint8_t* inKeyData,uint32_t inKeyDataLen,long inDataVal); |
| Functional description | S50 card value added |
| Input parameters | InBlock Block number 0-255 InKeyMod Key type 0=AKEY 1=BKEY inKeyDataLen Key data length InKeyData Key data inDataVal Increase amount |
| Output parameters | NULL |
| Return value | = 0 operation successfully < 0 operation failed |
| prerequisites | Hid_Init() succeeded |

## 2.9 PS_RF_DecValue

| Function name | PS_RF_DecValue |
| --- | --- |
| Function prototype | int32_t WINAPI PS_RF_DecValue(int32_t inBlock,int32_t inKeyMod,uint8_t* inKeyData,uint32_t inKeyDataLen,long inDataVal); |
| Functional description | S50 card Deduction amount |
| Input parameters | InBlock Block number 0-255 InKeyMod Key type 0=AKEY 1=BKEY inKeyDataLen Key data length InKeyData Key data inDataVal Deduction amount value |
| Output parameters | NULL |
| Return value | = 0 operation successfully < 0 operation failed |
| prerequisites | Hid_Init() succeeded |

## 2.10 PS_RF_BackUpValue

| Function name | PS_RF_BackUpValue |
| --- | --- |
| Function prototype | int32_t WINAPI PS_RF_BackUpValue(int32_t inSrcBlock,int32_t inTarBlock,int32_t inKeyMod,uint8_t* inKeyData,uint32_t inKeyDataLen); |
| Functional description | Backup wallet |
| Input parameters | inSector Source block 0-255 inTarBlock Target block 0-255 InKeyMod Key type 0=AKEY 1=BKEY inKeyDataLen Key data length InKeyData Key data |
| Output parameters | NULL |
| Return value | = 0 operation successfully < 0 operation failed |
| prerequisites | Hid_Init() succeeded |

## 2.11 PS_RF_CardSleep

| Function name | PS_RF_CardSleep |
| --- | --- |
| Function prototype | int32_t WINAPI PS_RF_CardSleep(); |
| Functional description | S50 card goes to sleep state |
| Input parameters | NULL |
| Output parameters | NULL |
| Return value | = 0 operation successfully < 0 operation failed |
| prerequisites | Hid_Init() succeeded |

# 3 Error return code

For debugging convenience, the error return value (non-zero return value) in the dynamic library is as follows:

#define	PARAMETER_ERR			-1001			//Parameter error

#define READ_ERR					-1002			//Read data error

#define WRITE_ERR					-1003			//Write data error

#define GET_HIDSTRUCT_ERR			-1004			//Get HID description error

#define HIDINIT_ERR					-1005			//Open RFID failed

#define HID_TIMEOUT				-1006			//Communication timeout

#define CRC_ERR						-1007			//Parity error

#define MODE_ERR					-1008			//Mode selection error

#define FORMART_ERR				-1009			//Packet format error

#define ZY_ERR1						-1000-0xE0		//Enter low power failure

#define ZY_ERR2						-1000-0xE1		//Open antenna failed

#define ZY_ERR3						-1000-0xE2		//Card search failed

#define ZY_ERR4						-1000-0xE3		//Card reading failed

#define ZY_ERR5						-1000-0xE4		//Card Writing failed

#define ZY_ERR6						-1000-0xE5		//Wallet initialization failed

#define ZY_ERR7						-1000-0xE6		//Read wallet balance failed

#define ZY_ERR8						-1000-0xE7		//Wallet recharge failed

#define ZY_ERR9						-1000-0xE8		//Wallet deduction failed

#define ZY_ERR10					-1000-0xE9		//Wallet backup failed

#define ZY_ERR11					-1000-0xEA		//Card operation failed

#define ZY_ERR12					-1000-0xEB		//Into automatic scan failed