Multifunctional module

manual

Version history

| version | Modify the description | Modifier | Modified date |
| --- | --- | --- | --- |
| 0.1 | Drafting |  | 2019-8-28 |

catalogue

1. Overview	3

1.1 Introduction to the Multifunction module	3

1.2 Feature	3

2. Software operation	4

2.1 Setting tool	4

2.2 MSR part	5

2.3 RFID part	12

2.4Fingerprint part	16

# 1. Overview

## 1.1 Introduction to the Multifunctional module

The Multifunctional module mainly includes four functions of magnetic card reader, IBUTTON, RFID and fingerprint.

## 1.2 Feature

### 1.2.1 MSR Feature:

- Support card reading in accordance with ISO7811, IBM,ANSI,JIS2,standards.

- Swipe speed：10-120cm/s.

- Life of magnetic head≥500000次.

- Readable magnetic strip coercive force range：300-4000 Oe.

- Magnetic strip density: track 1 210 BPI; track 2 75 ≤ 210 BPI; track 3 210BPI.

- Starter, separator, Terminator, track order are programmable.

- Optional configuration: 1 or 2 or 3 tracks; 1, 2 tracks; 2, 3 tracks; 1, 2, 3 tracks.

- Provides magnetic card setting software that sets the data format of the magnetic card.

### 1.2.2 Fingerprint Feature：

- Fingerprint comparison time speed：1:N < 1S (N≤200).

- Storage capacity: standard allocation of 96 fingerprint data.

- Sensor type: capacitive touch sensor.

- resolution ratio：508DPI.

- pixel：160*160.

- FRR<1%.

- FAR<0.001%.

- 8-bit grayscale.

- Service life: one million times.

### 1.2.3 RFID Feature：

- 1.	Support MifareOne S50, S70 and its compatible TYPE A, B card.

- 2.	Antenna and circuit board integration.

- 3.	The farthest read and write distance is more than 5cm.

# 2. Software operation

## 2.1 Test tools

<Figure 1> Set / Test Tool Launch Interface

### 2.2 MSR part：

Steps：

- Connecting equipment.

- Open the set/test tool, select “MSR Device”, and click “OK”. If the device is connected properly, go directly to the following settings interface; if the device is not connected properly, it will prompt “Can not find MSR Device”.

<Figure 2>Connection device success interface

- 3."ISO" setting tab: <Figure 3:>

<Figure 3> ISO setting interface

Emoticons:

“①”：Set the track to transmit the data.

“②”：Set the track for beginning sign.

“③”：Set the track for end sign.

“④”：Set to enable or disable carriage return.

“⑤”：Set return data head.<Definition method see Appendix>.

“⑥”：Set return data separator mark.<Definition method see Appendix>.

“⑦”：Set return data end.<Definition method see Appendix>.

“⑧”：“Enable Beep——Turn Beep on or off.

“Support IBM Card”——Support IBM card.

“Output Data For Any Valid Track”--Verify the track data valid.

“Reading Sequence”——Read track sequence.

“⑨”：Select the current system used language.

4."iButton" settings tab: <Figure 4:>

<Figure 4> iButton setting interface

Emoticons:

“①”：Set the iButton for beginning sign.

“②”：Set the iButton for end sign.

“③”：when“Attach”and“Remove” send data mode of iButton.

“④”：Set Attach of iButton return data head (ASCII code and KEYCODE code).

“⑤”：Set Remove of iButton return data head (ASCII code and KEYCODE code).

“⑥”：Set the data of Attach ,

“Extract Form” = start data bits

“Length” = data length

“Enable Beep” =when Attach  the Beep sound .

“⑦”：Set Attach of iButton return data end (ASCII code and KEYCODE code).

“⑧”：Set Remove of iButton return data end (ASCII code and KEYCODE code).

“⑨”：Set the data of Remove ,

“Extract Form” = start data bits

“Length” = data length

“Enable Beep” =when Remove the Beep sound .

5.Function buttons, as well as function buttons for the main menu: <Figure 5:>

<Figure 5>MSR function button interface

Emoticons:

“①”："Load" button: Read settings from MSR to DEMO.

“②”：“Normal Mode” button: Puts the MSR into test mode<only valid for ISO 			  format>.

“③”："Test Mode" button: Puts the MSR into test mode<only valid for ISO format>.

“④”："Save" button: Save the DEMO settings to the MSR.

“⑤”："Set Default" button: Set the MSR as the default parameter.

“⑥”："Test MSR" button: Magnetic card swipe test.

“⑦”： Exit the demo.

6.Help function menu: <Figure 6:>

<Figure 6> MSR version query button

Emoticons:

“①”："Version" menu, get the current version number of MSR.

“②”："About" menu for obtaining DEMO related information.

Appendix I:

ASCII way definition method

When the user defines "Header", "Separator", "Suffix" and "Prefix", the ASCII mode will be selected to enter the following interface.<Figure 7:>

<Figure 7> ASCII value entry interface

Emoticons:

- ①：The user enters the character directly in the edit box, and then clicks the “OK” button to define it successfully.

②：The definition of special function characters.

| Special characters | Definition method |
| --- | --- |
| Enter Key | \n or \N |
| ESC Key | \e or \E |
| Tab Key | \t or \T |
| \ Key | \\ |

③：The current version of the ascii string definition, the maximum length is 20.

Appendix II:

Code method definition method

When the user defines "Header", "Separator", "Suffix" and "Prefix", the CODE mode will be selected to enter the following interface.<Figure 8:>

<Figure 8> Key Code value entry interface

Emoticons:

- Click the button in the keyboard picture in the picture or directly enter it with the keyboard.

- The current version of the code is defined by a maximum length of 20 .

### 2.3 RFID part：

Steps:

- 1.Connecting equipment;

- Open the test tool, such as "Test Tool Start Interface" chart, select "RFID Device" and click the "OK" button. If the device is connected properly, it will enter the test interface directly. If the device is not connected properly, it will prompt "Can not find RFID Device".

<Figure 9> RFID test interface

- Place the test card near the antenna area, click the searchCard button, the information window prints the ID number of the test card, and the information window prompts “Search Card Successful”.

<Figure 10> Test card ID number

- RFID read and write function of MifareOne card.

<Figure 11> RFID read and write function module

This module is mainly used to read and write the MifareOne card and modify the key:

Example：

Reading card:

For example, read the second block in sector 8

- Select the 8th sector from the "①" of the " read/write module" picture, and select the 2nd block in the "②" in the figure.

- Select a key from the "③" of the "RFID read/write module" picture.

- Enter "FFFFFFFFFFFF" 12 F in the "④" of the "RFID Read/Write Module" picture. For all new cards, the default key is 12 F.

④  Click the "Read Card" button on the "RFID Read/Write Module" picture to read the card.

Note: For the third block of each sector, 16 bytes are divided into 3 parts, the first 6 bytes are “A key”, and 	the middle 4 bytes are “key control word” followed by 6 bytes. B key".

Write card：

For example:write 16 bytes "0123456789ABCDEFFEDCBA9876543210" to the second block in the 8th 	sector

- Select the 8th sector from the "①" of the "RFID read/write module" picture, and select the 2nd block in the "②" in the figure.

- Select the A key from the "③" in the "RFID read/write module" picture.

- Enter "FFFFFFFFFFFF" 12 F in the "④" of the "RFID Read/Write Module" picture. For all new cards, the default key is 12 F.

- Enter "0123456789ABCDEFFEDCBA9876543210" in the third edit box of "⑤" in the "RFID Read/Write Module" picture.

- Click the "Write Card" button in the "RFID Read/Write Module" picture to read the card.

Note: To write the third block of each sector, special care must be taken. The third block holds the key for each sector. Once this block is written, the sector key will be modified.

Modify the key：

For example: change the key of the 8th sector from the original "FFFFFFFFFFFF" to "888888888888"

- Select the 8th sector from the "①" of the "RFID read/write module" picture, and select the 3rd block in the "②" in the figure.

- Select the A key from the "③" in the "RFID read/write module" picture.

- Enter "FFFFFFFFFFFF" 12 F in the "④" of the "RFID Read/Write Module" picture.

- Click the "Read Card" button in the "RFID Read/Write Module" picture to read the card. If the card is successfully read, "000000000000FF078069FFFFFFFFFFFF" will be displayed in the fourth edit box of "⑤" in the "RFID Read/Write Module" diagram.

"000000000000 FF078069 FFFFFFFFFFFF" (The first 6 bytes of A key are displayed as 12 "0"s, but actually 12 internal "F", which is displayed as 0 for confidentiality, and the next 6 bytes are dense. The key shows 12 "F", because the default B key in the factory card is not used for authentication, so it can be read).

- Change the first 6 bytes of "000000000000" in the fourth edit box to "888888888888".

⑥　Click the "Write Card" button in the "RFID Read/Write Module" picture to read the card. After the 　　　　　　　　　　　		prompt is successful, the key of the 8th sector of the card is modified to 12 "8" or "888888888888".

Note: Pay special attention when changing the password. First, remember the new password. Second, do 	not change the control word easily. Otherwise, the sector card may not be read or written.

- MifareOne card wallet function：

<Figure 12> RFID wallet function

This module is mainly used to use a piece of MifareOne card as an electronic wallet.

For example: use the 0th block of the 1st sector of the MifareOne card as an electronic wallet, and perform the following operations on the wallet: initializing, recharging, debiting, and checking the balance operation.

- Select the first sector from the "①" of the "RFID wallet function" picture, and select the 0th block in the "②" in the figure.

- Select the A key from the "③" in the "RFID wallet function" picture.

- Enter "FFFFFFFFFFFF" 12 F in the "④" of the "RFID Wallet Function" picture. For all new cards, the default key is 12 F.

- Enter the amount in the “⑤” of the “RFID Wallet Function” picture. At this time, the amount of the value entered in “⑤” will be automatically displayed in the “⑥” in the figure.

- Initialize the wallet of the 0th block of the 1st sector to 88: Enter 88 in the "⑤" of the "RFID wallet function" picture, and then click the "Initialize" button.

- Recharge 12 dollars into the wallet: Enter 12 in the “⑤” of the “RFID Wallet Function” picture, then click the “Increase” button.

- Deduct 2 from the wallet: Enter 2 in the "⑤" of the "RFID wallet function" picture, and then click the "Decrease" button.

- Check the balance: Click the “Balance” button, which will display 98 in the “5” of the “RFID Wallet Function” picture.

### 2.4 Fingerprint part：

Steps:

- Connecting device

- Open the test tool, such as "Fingerprint Device" in the "Test Tool Launch Interface" 	diagram, click the "OK" button to enter the fingerprint interface interface test interface.

<Figure 13> Fingerprint test interface

- Turn the fingerprint device on and off:

<Figure 14> The fingerprint reader is turned on, off, and exited

Emoticons:

①：Click on the “①Open device” button in the figure, you will try to open the fingerprint device. If it is successfully opened, it will prompt“Open the device successfully”.

②：Click the “②Close device” button in the figure to turn off the fingerprint device.

③：Click the “③Quit” button in the figure to exit the fingerprint test interface.

- Enter and verify fingerprint functions:

<Figure 15> Fingerprint input fingerprint and verification fingerprint function

Emoticons:

①：Click the “①Enroll fingerprint” button and the fingerprint will start to be entered.Note that the value entered in the edit box below the "Enter the ID of enroll finger" is <0 to 95>. If you enter the fingerprint, you need to put the finger 4 times. If you enter the fingerprint, To exit the entry, click on the "③Cancel operation" button.

②：Click the “②Verify fingerprint” button to verify the user's fingerprint.Compare the current fingerprint of the user with the fingerprint in the fingerprint database. If successful, it will return the ID number in the fingerprint database where the current fingerprint is located.

- Fingerprint library related operations:

<Figure 16> Fingerprint instrument input fingerprint library related operations

Emoticons:

①：Click the “①Refresh user ID table” button, the fingerprint ID number that already exists will be read from the fingerprint database of the fingerprint reader and saved in the fingerprint library list on the left.

②：In the fingerprint library list on the left, select any ID and click the “②Delete specified ID” button to delete the corresponding ID from the fingerprint library.

③：Click the “③Clear all IDs” button to clear the fingerprint library in the fingerprint reader.