- package name:cn.wintec.wtandroidjar

- Cash Drawer

import cn.wintec.wtandroidjar.Drw;

Drw drw=null;

String devicePath="/dev/ttySAC1";

drw=new Drw(devicePath);

| No. | Function Description | Function | remark |
| --- | --- | --- | --- |
| 1 | open cash drawer | boolean DRW_Open() |  |
| 2 | Gets Cash drawer status | boolean DRW_GetStatus (byte[] status) |  |
| 3 | Close cash drawer | void DRW_Close() |  |

- Customer Display (CD110)

import cn.wintec.wtandroidjar.Dsp110;

Dsp110 dsp=null;

String devicePath="/dev/ttySAC3";

dsp =new Dsp110 (devicePath);

| No. | Function Description | Function | remark |
| --- | --- | --- | --- |
| 1 | Clear screen: Clears all characters on the screen. | boolean DSP_ClearScreen() |  |
| 2 | Clear cursor line: Clear cursor line character, move the cursor to the right. | boolean DSP_ClearLine() |  |
| 3 | Initialization: Display "多谢惠顾" “ | boolean DSP_Init() |  |
| 4 | Display Data | boolean DSP_Dispay(String data,String charset) | data: Data to be displayed charset: cp866 cyrillic |
| 5 | Set cursor state | boolean DSP_SetCursorState(int state) | state:1-bright; 0- dark |
| 6 | Move the cursor position:move the cursor to “pos” position | boolean DSP_MoveCursor(int pos) | pos 1<=pos<=8 |
| 7 | Settings "收款", "单价", "合计", "找零" display status | boolean DSP_SetWords1State(int state) | state 0<=state<=4 |
| 8 | Setting "多谢惠顾" character display state | boolean DSP_SetWords2State(int state | state:1-bright; 0- dark |
| 9 | Brightness adjustment | boolean DSP_SetBrightness(int level) | level 0<=level<=7 |
| 10 | Close customer display | void DSP_Close() |  |

- Customer Display(pos card, Compatible cd320 cd220)

import cn.wintec.wtandroidjar.DspPos;

DspPos dsp=null;

String devicePath="/dev/ttySAC3";

dsp =new DspPos (devicePath);

| No. | Function Description | Function | remark |
| --- | --- | --- | --- |
| 1 | Set the baud rate | boolean DSP_SetBaudRate(int baudrate) |  |
| 2 | Set Chinese / English display mode | boolean DSP_SetDspMode(int n) | n: 0-chinese;1-english |
| 3 | Cursor left | boolean DSP_MoveCursorLeft() |  |
| 4 | Cursor right | boolean DSP_MoveCursorRight() |  |
| 5 | Cursor down | boolean DSP_MoveCursorDown() |  |
| 6 | Cursor up | boolean DSP_MoveCursorUp() |  |
| 7 | Cursor homing | boolean DSP_CursorHome() |  |
| 8 | Move the cursor to the leftmost position of the current line | boolean DSP_MoveCursorEndLeft() |  |
| 9 | Move the cursor to the right-most position of the current row | boolean DSP_MoveCursorEndRight() |  |
| 10 | Move the cursor to the right-most position or end-line | boolean DSP_MoveCursorEnd() |  |
| 11 | Move the cursor to the specified location | boolean DSP_MoveCursor(int m, int n) | m:line n:column |
| 12 | Clear all characters on the screen | boolean DSP_ClearScreen() |  |
| 13 | Clear cursor line character, move the cursor to the leftmost | boolean DSP_ClearLine() |  |
| 14 | Initialize the display | boolean DSP_Init() |  |
| 15 | Line display the data | boolean DSP_DisplayLine(int n,String data,String charset) | n:line,1or2 data: Data to be displayed charset: cp866 cyrillic |
| 16 | Select the standard ASCII characters set | boolean DSP_SetASCII(int n) | 0≤n≤15 |
| 17 | Select the extended character set | boolean DSP_SetExtendASCII(int n) | 0≤n≤15 |
| 18 | Set the display mode | boolean DSP_DisplayMode(int n) | n: 1 - Cover;2 - Vertical; 3 - Horizontal |
| 19 | Character blinking frequency setting | boolean DSP_SetBlink(int n) | 0≤n≤FFHex |
| 20 | Self-test | boolean DSP_SelfTest() |  |
| 21 | Download custom character set | boolean DSP_UploadChars(String path,int dwArea) | ? |
| 22 | Contrast Adjustment | boolean DSP_SetContrast(int n) | n:1-40%;2-60%;3- 80%; 4-100% |
| 23 | Display Data | boolean DSP_Dispay(String data,String charset) | data: Data to be displayed charset: cp866 cyrillic |
| 24 | Close customer display | void DSP_Close |  |

- IButton

- import cn.wintec.wtandroidjar.IButton;

IButton ibutton=null;

String devicePath="/dev/ ttySAC3";

ibutton =new IButton(devicePath);

| No. | Function Description | Function | remark |
| --- | --- | --- | --- |
| 1 | Get the serial number | int IBTN_GetSN(byte[] bytes) | bytes: serial number return:Serial Number Length |
| 2 | Write data | boolean IBTN_Write(byte[] data) |  |
| 3 | Read data | boolean IBTN_Read(byte[] data) |  |
| 4 | Close ibutton | void IBTN_Close() |  |

- Magnetic Card Reader

cn.wintec.wtandroidjar.Msr;

Msr msr=null;

String devicePath="/dev/ttySAC3";

msr = Msr (devicePath);

| No. | Function Description | Function | remark |
| --- | --- | --- | --- |
| 1 | Read track data | String MSR_GetTrackData(byte enterTrack1,byte enterTrack2,byte enterTrack3 ) | enterTrackn:Enter identify whether there |
| 2 | Close a track | boolean MSR_EnableTracks(int n) | 1≦n≦3 |
| 3 | Open a track | boolean MSR_DisableTracks(int n) | 1≦n≦3 |
| 4 | Set start character for a track | boolean MSR_SetStartSentinel(int n,char ch) | n:track number(1≦n≦3) ch:start character |
| 5 | Set end character for a track | boolean MSR_SetEndSentinel(int n,char ch) | n:track number(1≦n≦3) ch:end character |
| 6 | Open sound | boolean MSR_EnableBeep() |  |
| 7 | Close sound | boolean MSR_DisableBeep() |  |
| 8 | Suport IBM | boolean MSR_EnableIBM() |  |
| 9 | Don’t suport IBM | boolean MSR_DisalbeIBM() |  |
| 10 | Set the reading order of tracks | boolean MSR_ReadSequence(int s1, int s2, int s3) | Three track card reader order ,parameter can not be repeated |
| 11 | Get MSR configuration | int MSR_GetConfiguration(byte[] configbytes) |  |
| 12 | Write data | boolean MSR_Write(byte [] content) |  |
| 13 | Close MSR | void MSR_Close() |  |

- Printer(COM port)

cn.wintec.wtandroidjar.Printer;

Printer printer=null;

String devicePath="/dev/ttySAC1";

printer=new wintePrinter(devicePath);

| No. | Function Description | Function | remark |
| --- | --- | --- | --- |
| 1 | Initialize the printer | boolean PRN_Init() |  |
| 2 | Close the printer | void PRN_Close() |  |
| 3 | Get paper status | boolean PRN_GetPaperStatus() | return true:has paper false:no paper |
| 4 | Set horizontal positioning | boolean PRN_SetHT(byte[] bytes) |  |
| 5 | Perform horizontal positioning | boolean PRN_HT() |  |
| 6 | Print data in the buffer | boolean PRN_PrintBuffer() |  |
| 7 | Print and feed n points | boolean PRN_PrintAndFeedDot(int n) |  |
| 8 | Print and feed n lines | boolean PRN_PrintAndFeedLine(int n) |  |
| 9 | Set right-side character spacing | boolean PRN_SetCharRightSpace(int n) | 0<=n<=255 |
| 10 | Select print mode | boolean PRN_SelectPrintMode(int n) | 0<=n<=255 |
| 11 | Set the default spacing | boolean PRN_SetDefaultSpace() |  |
| 12 | Set line spacing | boolean PRN_SetLineSpace(int n) |  |
| 13 | Set /cancel heavier Print | boolean PRN_EnableBoldFont(int n) | n:0-cancel;1-bold |
| 14 | Select the character set of nationality | boolean PRN_SelectUnicode(int n) | n: 0-13 United States, France, Germany, Britain, Denmark I, Sweden, Italy, Spain, I, Japan, Norway, Denmark II, Spain II, Latin America, South Korea |
| 15 | Open the Paper Feed button | boolean PRN_EnableFeedKey() |  |
| 16 | Close the Paper Feed button | boolean PRN_DisableFeedKey() |  |
| 17 | enable double width and height | boolean PRN_EnableDoubleWidthandHeight() |  |
| 18 | disable double width and height | boolean PRN_DisableDoubleWidthandHeight() |  |
| 19 | enable font double width | boolean PRN_EnableFontDoubleWidth() |  |
| 20 | disable font double width | boolean PRN_DisableFontDoubleWidth() |  |
| 21 | enable font double height | boolean PRN_EnableFontDoubleHeight() |  |
| 22 | disable font double height | boolean PRN_DisableFontDoubleHeight() |  |
| 23 | Multiple font size | boolean PRN_FontMultiple(int n) | 0<=n<=6 0-normal |
| 24 | enable underline | boolean PRN_EnableFontUnderline() |  |
| 25 | disable underline | boolean PRN_DisableFontUnderline() |  |
| 26 | enable chinese mode | boolean PRN_EnableChinese() |  |
| 27 | disable chinese mode | boolean PRN_DisableChinese() |  |
| 28 | Set around the Chinese character spacing | boolean PRN_SetChineseSpace(int n1, int n2) |  |
| 29 | Cut paper | boolean PRN_CutPaper() |  |
| 30 | Half cut paper | boolean PRN_HalfCutPaper() |  |
| 31 | Print string | boolean PRN_Print(String data,String charset) | charset: ASCII/UTF-8 english GBK chinese ISO8859-1 Latin-1 ISO8859-2 Latin-2 ISO8859-3 Latin-3 ISO8859-4 Latin-4 ISO8859-5 Cyrillic ISO8859-6 Arabic ISO8859-7 Greek ISO8859-8 Hebrew ISO8859-8-I Hebrew ISO8859-9 Latin-5 or Turkish ISO8859-10 Latin-6 or Nordic ISO8859-11 Thai ISO8859-13 Latin-7 or Baltic Rim ISO8859-14 Latin-8 or Celtic ISO8859-15 Latin-9 ISO8859-16 Latin-10 |
| 32 | Print string | boolean PRN_Print(String data,String charset,boolean isEnd) | isEnd true:current line is finished False:current line not finish |
| 33 | Print 1 bit bmp | boolean PRN_PrintDotBitmap(String filePath,int n) | n:0-left alignment;1-center alignment;2-right alignment |
| 34 | Print 1 bit bmp | boolean PRN_PrintDotBitmap(Bitmap bitmap,int n) | n:0-left alignment;1-center alignment;2-right alignment |
| 35 | Print 1 bit bmp once | boolean PRN_PrintDotBitmap1(String filePath,int n,int size) | n alignment mode.( Unrealized) 0-left,1-center,2-right size picture size 0-nomal ,1-double wdth,2-double height,3-four times |
| 36 | Print 1 bit bmp once | boolean PRN_PrintDotBitmap1(Bitmap bitmap,int n,int size) | n alignment mode.( Unrealized) 0-left,1-center,2-right size picture size 0-nomal ,1-double wdth,2-double height,3-four times |
| 37 | define NV bmp | boolean PRN_DefineNVBmp(String filePath) | the width and height of bmp file must be in multiples of 8 |
| 38 | define NV bmp | boolean PRN_DefineNVBmp(Bitmap bitmap) | the width and height of bmp file must be in multiples of 8 |
| 39 | print NV bmp | boolean PRN_PrintNVBitmap(int n) | n 0:normal 1:double width 2:double height 3:four times |
| 40 | Set barcode HRI position | boolean PRN_SetBarCodeHRI (int bHRI) | bHRI - don't print ; - top; - bottom; - top and bottom |
| 41 | Set barcode print height | boolean PRN_SetBarCodeHeight(int n) | 1<=n<=255 |
| 42 | Set barcode print width | boolean PRN_SetBarCodeWidth(int n) | 2<=n<=6 |
| 43 | Print barcode | boolean PRN_PrintBarCode(int m, String barcode) | 0<m<6 - UPC-A - UPC-E - JAN13(EAN13) - JAN8(EAN8) - CODE39 - ITF - CODABAR 65<m<73 - UPC-A - UPC-E - JAN13(EAN13) - JAN8(EAN8) - CODE39 - ITF - 35CODABAR |
| 44 | Set code page | boolean PRN_SetCodePage(int n) | n 0:PC437(Std. Europe) 1:(Catakana) 2:PC850(Multilingual) 3:PC860(Portuguese) 4:PC863(Candian) 5:PC865(Nordic) 6:(West Europe) 7:(Greek) 8:(Hebrew) 9:(Ease Europe) 10:(Iran) 16:(WPC1252) 17:PC866(Cyrillic#2) 18:PC852(Latin2) 19:PC858(Euro) 20:(IranII) 21:(Latvian) 22:(Arabic) 23:(PT152,1251) 24:(PC747) 25:(WPC1257) 27:(Vietram) 28:(PC864) 29:(PC1001) 30:(Uigur) 31:(Hebrew) 32:WPC1255(Israel) 255:(Thai) 50:PC437(Std.Europe) 51:(Katakana) 52:PC437(Std.Europe) 53:PC858(Multilingual) 54:PC852(Latin-2) 55:PC860(Portuguese) 56:PC861(IceLandic) 57:pc863(Canadian) 58:PC865(Nordic) 59:PC866(Russian) 60:PC855(Bulgarian) 61:PC857(Turkey) 62:PC862(Hebre) 63:PC864(Arabic) 64:PC737(Greek) 65:PC851(Greek) 66:PC869(Greek) 67:PC928(Greek) 68:PC772(Lithuanian) 69:PC774(Lithuanian) 70:PC874(Thai) 71:WPC1252(Latin-1) 72:WPC1250(Latin-2) 73:WPC1251(Cyrilic) 74:PC3840(IBM_Russian) 75:PC3841(Gost) 76:PC3843(Polish) 77:PC3844(CS2) 78:PC3845(Hungarian) 79:PC3846(Turkisn) 80:PC3847(Brazil_ABNT) 81:PC3848(Brazil-ABICOMP) 82:PC1001(Arabic) 83:PC2001(Lithuanian-KBL) 84:PC3001(Estonian-1) 85:PC3002(Estonian-2) 86:PC3011(Latvian-1) 87:PC3012(Latvian-2) 88:PC3021(Bulgarian) 89:PC3041(Maltese) 90:PC1254(Turkish) 91:PC1257(Baltic) 92:PC1256(Arabic) |

- Printer(usb port)

Function is as above(6.Printer(COM port))

import cn.wintec.wtandroidjar.UsbPrinter;

UsbPrinter printer = new UsbPrinter(1155,30016);

- RFID

import cn.wintec.wtandroidjar.Rfid;

Rfid rfid;

String devicePath="/dev/ ttySAC3";

rfid=new Rfid(devicePath);

| No. | Function Description | Function | remark |
| --- | --- | --- | --- |
| 1 | Set the module non-contact work mode | boolean RFID_SetWorkWay(int type) |  |
| 2 | Search card | boolean RFID_SearchCards(byte[] cardtype) | Return card type |
| 3 | Anti-collision | boolean RFID_AntiCollision(byte[] cardNumber) | Return 4 bytes card serial number |
| 4 | Select card | boolean RFID_SelectCard(byte [] buffer,byte[] Capacity) | buffer:4 bytes card serial number capacity:Card Capacity |
| 5 | Authentication Key | boolean RFID_VerifyKey(int block,byte[] bufferKey,byte cPwdFlags) | block:block number bufferKey: key cPwdFlags:key type |
| 6 | Read block | Int RFID_ReadBlock(int num,byte [] cardvalue) | num:block number cardvalu:block data return:data length |
| 7 | Write block | boolean RFID_WriteBlock(int num, byte[] buffer) | num:block number buffer:data to be written |
| 8 | Initialize Wallet | boolean RFID_InitWallet(int num, int money) | num:block number money:initialize money |
| 9 | Read wallet | boolean RFID_ReadWallet(int num, byte [] money) | num:block number money: balance |
| 10 | Recharge | boolean RFID_RechargeWallet( int num,int money) | num:block number money: recharge amount |
| 11 | Deduct money | boolean RFID_DeductMoney(int num, int money) | num:block number money: deduct amount |
| 12 | Card Halt | boolean RFID_Halt() |  |
| 13 | set baudrate | boolean RFID_SetBaudrate(int baudrate) |  |
| 14 | Open Antenna | boolean RFID_EnableAntenna() |  |
| 15 | Close antenna | boolean RFID_DisableAntenna() |  |
| 16 | CPU card special-ISO14443 TYPE A CPU card detecting card and reset | boolean RFID_Pro_Reset(byte mode,byte[] rlen, byte[] receive_data) | mode:Search card mode (0x26 Request not enter hibernation card; 0x52 cards to find all states) rlen: The length of the returned data receive_data: Return data (4 bytes CSN + Reset content) |
| 17 | CPU card special - COS instruction execution | boolean RFID_Cos_Command(int clen, byte[] command_data, byte[] rlen, byte[] receive_data) | clen:Command length command_dat:Command data rlen:Length of the data returned by the card receive_data: the data returned by the card |
| 18 | Close RFID | void RFID_Close() |  |

- GPIO

import cn.wintec.wtandroidjar.GPIO;

GPIO gpio= new GPIO();

| No. | Function Description | Function | remark |
| --- | --- | --- | --- |
| 1 | Set GPIO output state | void setValue( int value) | value:0-output low;1-output high |
| 2 | Get GPIO input status | int getValue() | return: 0-Input is low; 1-Input is high |

- uart operation

import cn.wintec.wtandroidjar.ComIO;

ComIO comio=null;

String devicePath="/dev/ttySAC?";

comio = ComIO (devicePath);

| No. | Function Description | Function | remark |
| --- | --- | --- | --- |
| 1 | Write data to the uart | void write(String string) |  |
| 2 | Write buffer contents to the UART | void write(byte[ ] buffer) |  |
| 3 | Read data from the uart | int read(byte[] buffer, int offset, int length) | buffer: the buffer of store read data offset: the start position data stored in the buffer length: Maximum read length return: The actual number of bytes read. If the device does not exist, returns -1. |
| 4 | read data from the uart,overtime | int read(byte[] buffer, int offset, int length, long millisecond) | buffer: the buffer of store read data offset: the start position data stored in the buffer length: Maximum read length millisecond:timeout return: The actual number of bytes read. If the device does not exist, returns -1 |
| 5 |  | void readFinish() | If an object is called off ComIO read function, be sure to call this function recovery resources; This function also cancel're reading read function |
| 6 | Set UART parameter | void setSerialParam( Baudrate baudrate, VerifyBit verifyBit, DataBit dataBit, StopBit stopBit, FlowControl flowControl) |  |
| 7 | Get UART parameter | Param getSerialParam() |  |
| 8 | Get UART baudrate | Baudrate getSerialBaudrate() |  |
| 9 | Set UART baudrate | void setSerialBaudrate(Baudrate baudrate) |  |
| 10 | Set UART baudrate | void setSerialBaudrate(int baudrate) |  |

- System Information

import cn.wintec.wtandroidjar.SYS;

SYS sys=new SYS();

| No. | Function Description | Function | remark |
| --- | --- | --- | --- |
| 1 | Read the board's default Ethernet MAC address | String getDefaultMAC() | return:The default Ethernet MAC address, for example12:34:56:78:9A:BC * If it has not set the default address, returns an empty string |
| 2 | Modifying the default Ethernet MAC address of the board | boolean setDefaultMAC(String mac) | mac:The default Ethernet MAC address, for example12:34:56:78:9A:BC, Change of Address take effect after the system restarts |