VM3290 Setting Manual

Version

| Version NO. | Version description | Issue date |
| --- | --- | --- |
| V1.0.0 | Initial version | 2019-10-24 |

Connect to scanner	9

Enable/Disable configuration code	9

Version	10

Restore factory setting	10

User configuration	11

Interface mode	11

USB keyboard configuration	12

Enter and LF settings (USB keyboard)	13

Transmission speed （USB keyboard）	14

Letter case output (USB keyboard)	15

Keyboard Layouts	15

Virtual keyboard	20

Select host operating system in virtual keyboard mode	21

Output coded format	22

Invoice function	23

RS232 serial port configration	24

Serial port baud rate configuration	24

Serial port data bit, stop bit, check bit configuration	25

GS control character replacement	27

Scan mode	29

Auto sense mode	29

Detecting same barcodes	29

Center mode	30

Indicator light	31

Decoded success LED Indicator light	31

Buzzer	32

Volume	32

Scanner power on tone	32

Decoded success beep	33

Decoded success beep voice frequency	33

Decoded success beep duration	34

False warning beep voice frequency	34

Prefix and suffix	35

Start character	35

Ending character	36

Customize prefix	37

Customize suffix	38

Code ID	39

AIM ID	41

Barcode prefix and suffix order selection	41

Prefix	41

Suffix	42

Data editing	42

Transmission configuration	43

Field length configuration	44

Reversed barcode option	45

Barcode type selection	46

Trun on / off all barcodes type	46

Trun on / off all 1D barcodes type	46

Trun on / off all 2D barcodes type	47

Codabar	47

Codabar Start character/end character	48

Codabar Length limit setting	48

Code 39	49

Code 39 check bit	49

Code 39 Full ASCII	50

Code 39 Length limit setting	50

Code 32  (code39 needs to be turned on)	51

Interleaved 2 of 5（ITF25）	51

Interleaved 2 of 5（ITF25）check bit	52

Interleaved 2 of 5（ITF25）length selection	53

Interleaved 2 of 5 Length limit setting	55

Industrial 2 of 5（Industrial 2 of 5）	55

Industrial 2 of 5 Length limit setting	56

Matrix 2 of 5（Matrix 2 of 5）（4-24 bits）	56

Matrix 2 of 5 Length limit setting	57

Code 93	57

Code 93 Length limit setting	58

Code 11	58

Code 11 Check bit output	59

Code 11 check bit selection	60

Code 11 Length limit setting	60

Code 128	61

ISBT-128	61

GS1-128	62

GS1-128 Length limit setting	62

UPC-A	63

UPC-A check bit	63

UPC-A convert to EAN-13	63

UPC-E	64

UPC-E check bit	65

UPC-E extend to UPC-A	65

EAN/JAN-8	65

EAN/JAN-13	66

UPC/EAN/JAN extracode	66

EAN13 convert to ISBN	67

EAN13 convert to ISSN	67

GS1 DataBar（RSS14）	68

GS1 DataBar Limited	68

GS1 DataBar Expanded	69

MSI code	69

MSI check bit output	70

MSI check bit selection	70

MSI code 2 bits check bit selection	71

MSI code length limit setting	71

Febraban（ Brazil bank document barcode）	71

Febraban（ITF25）	72

Febraban（CODE128）	72

Febraban parity	72

PDF417	73

Micro PDF417	73

QR Code	74

Micro QR	74

Data Matrix	75

Aztec Code	75

Appendix	76

Data code	76

Bar code type ID table	80

AIM  ID table	81

ASCII table	82

Control character set (USB keyboard)	83

Control character set（serials port and USB-COM）	84

Some function configuration instructions and examples	86

Custom suffix configuration example	86

Barcode Length Limit Configuration Example	87

USB keyboard send speed configuration example	88

Warning sound	88

Reading tips	88

Safety	90

Connect to scanner

Connect the scanner to the connector on the cable and the other end to the corresponding port on the PC.

If the barcode configuration function has been turned off, turn on the barcode configuration function by scanning the setup code.

Enable/Disable configuration code

When the configuration code function is turned on, all configuration barcodes can be scanned for scanner configuration.

When the configuration code function is turned off, other configuration barcodes cannot be scanned for scanner configuration. You need to reopen to scan the configuration code for scanner configuration.

Enable configuration code*

Disable configuration code

Version

Version number

Restore factory setting

Restore factory setting

User configuration

Scan the barcode below to save the current parameters of the product as a user configuration.

Save user configuration

Scan the barcode below to return the product to a saved user configuration.

Restore user configuration

Interface mode

This device supports USB keyboard, USB to serial port (USB-COM), serial port interface mode.

USB mode *

RS232 mode (serial port)

USB-COM mode (driver is needed)

USB keyboard configuration

Turn on control character escape

Turn off control character escape *

Enter and LF settings (USB keyboard)

Enable 0A (LF)

Enable 0D（Enter CR）*

Enable 0A (LF) + 0D (CR)

Transmission speed （USB keyboard）

Used to configure the speed when sending data in USB keyboard mode. If the PC you are using has low performance, it is recommended to choose a low speed to ensure transmission accuracy.

Low speed *

Medium speed

High speed

Custom transmission speed（2ms~50ms）

Letter case output (USB keyboard)

Normal output *

Letter case reversal

All Caps

All lowercase

Keyboard Layouts

English (United States)  *

French (France)

Italian (Italy)

Italian 142 (Italy)

German (Germany)

Spanish (Spain)

Spanish (Latin America)

Finnish

Japanese

Russian (MS)

Russian (typewriter)

Arabic (101)

Irish

Polish (214)

Polish (Programmers)

Dutch (Netherlands)

Czech (QWERTZ)

Portuguese (Portugal)

Portuguese (Brazil)

Swedish (Sweden)

Q  Turkish Q

Turkish F

Greek (MS)

French (Belgium)

English (UK)

Virtual keyboard

Mode 1： Characters between 0x20 and 0xFF are not supported under the current keyboard layout and are output using the virtual keyboard. Characters between 0x00 and 0x1F are output according to the control character definition（Refer to appendix）

Mode 2：All characters between 0x20 and 0xFF are output using the virtual keyboard. Characters between 0x00 and 0x1F are output according to the control character definition (Refer to appendix)

Mode 3：All characters used between 0x00 and 0xFF are output using the virtual keyboard.

Turn off virtual keyboard *

Turn on virtual keyboard（mode 1）

Turn on virtual keyboard（mode 2）

Turn on virtual keyboard（mode 3）

Select host operating system in virtual keyboard mode

WINDOWS *

MAC OS

LINUX

Output coded format

In order to output correctly according to the specified encoding format, you need to specify the output encoding format, such as Simplified Chinese in Notepad / excel output configuration into GBK encoding, in Word and other output configured into UNICODE encoding.

When the output encoding format is configured as English/Latin-1 encoding, the output mode of the USB keyboard is affected by the virtual keyboard function switch. When the output encoding format is configured as GBK encoding / UNICODE encoding, the output mode of the USB keyboard is forced to be the virtual keyboard output.

English/latin-1 encoding *

GBK code (notepad /excel)

UNICODE encoding (Word)

Invoice function

Disable Invoice function *

Enable Invoice function

To ensure the correct output of the invoice content, when opening the invoice code function, please configure the Chinese character output mode to GBK code (Notepad/excel), and turn off the function of changing the original content of the barcode, such as CodeID, custom pre/suffix, and start character.

Invoice type selection

Special invoice *

Regular invoice

RS232 serial port configration

Serial port baud rate configuration

Baud rate 4800

Baud rate 9600 *

Baud rate 19200

Baud rate 38400

Baud rate 57600

Baud rate 115200

Serial port data bit, stop bit, check bit configuration

7 data bit, 1 stop bit, no check bit

7 data bit, 1 stop bit, check even number

7 data bit, 1 stop bit, check odd number

7 data bit, 2 stop bit, no check bit

7 data bit, 2 stop bit, check even number

7 data bit, 2 stop bit, check odd number

8 data bit, 1 stop bit, no check bit *

8 data bit, 1 stop bit, check even number

8 data bit, 1 stop bit, check odd number

8 data bit, 2 stop bit, no check bit

8 data bit, 2 stop bit, check even number

8 data bit, 2 stop bit, check odd number

GS control character replacement

Not to replace *

To output the character "Ç", you must first scan "enable virtual keyboard (mode 1)" or (mode 2) or (mode 3)

Repalce with Ç

Repalce with |

Repalce with  ^]

Repalce with ]

Repalce with    <GS>

Scan mode

Auto sense mode

Auto sense off *

Auto sense on

Detecting same barcodes

It is used to configure the interval time for solving the same bar code. If it does not exceed the set time, the same bar code will only be solved once.

Interval time 500ms

Interval time 750ms *

Interval time 1s

Interval time 2s

Center mode

When the center mode is turned on, the scanner will only recognize the bar code located in the center area of the picture facing the scanner lens. This configuration is off by default。

Center mode OFF *

Center mode ON

Indicator light

Decoded success LED Indicator light

OFF

ON *

Buzzer

Volume

Low

High *

Scanner power on tone

OFF

ON *

Decoded success beep

OFF

ON *

Decoded success beep voice frequency

voice frequency 1 *

voice frequency 2（2.7K）

voice frequency 3

Decoded success beep duration

Long *c

Short

False warning beep voice frequency

When the data transmission fails, the code scanning gun will issue four error warning sounds. When it scans an unrecognized configuration code, the code scanning gun will issue a single error warning sound.

Low *

Intermediate

High

Prefix and suffix

Start character

NO start character *

Set start character as STX

Ending character

No ending Character

CR

LF

CR+LF *

TAB

ETX

Customize prefix

Output options

Enable customized prefix output

Enable customized prefix output *

Configuration

Remove all customized prefix

Customize prefix

（After scanning, please set according to the barcode type ID table and data and edit the barcode in the appendix.）

Customize suffix

Output options

Enable customized suffix output

Disable customized suffix output *

Configuration

Remove all customized suffix

Customize suffix

Code ID

Output options

Turn off CODE ID *

Turn on front CODE ID

Turn on ending CODE ID

Configuration

Customize CODE ID

（After scanning, please set according to the barcode type ID table and data and edit the barcode in the appendix.）

Remove all customized CODE ID

AIM ID

Turn off AIM ID *

Turn on front AIM ID

Turn on ending AIM ID

Barcode prefix and suffix order selection

Prefix

Start character +CODE ID+AIM ID+ customized prefix *

Start character+ customized prefix+CODE ID+AIM ID

Suffix

Customized suffix+CODE ID+AIM ID+ending character *

CODE ID+AIM ID+ Customized suffix + ending character

Data editing

The data editing function can customize the Data field of the complete barcode content into 3 fields of Start / Center / End by configuring the Start / End field length. Please configure the length and transmission configuration of the Start / End field according to actual needs.

Note: Custom content such as custom suffix, start character, end character, CODE ID, AIM ID are not affected by data editing

Transmission configuration

Transmit full data *

Only transmit start field of barcode

Only transmit center field of barcode

Only transmit ending field of barcode

Field length configuration

Set start field length

Set ending field length

Note: The field length configuration is in bytes and is configured using decimal data.

For example: Set the length of the Start segment to 10 bytes, scan the barcode for setting the length of the Start segment, and then scan the data in the appendix and edit the 1, 0 in the barcode to save the configuration.

Reversed barcode option

（1D barcode only /DataMatrix/Aztec）

Only read normal barcodes

Only read reversed barcodes

Read normal barcodes and reversed barcodes

Barcode type selection

Trun on / off all barcodes type

Opening all bar code types may result in slower decoding speed. It is recommended to open the required bar code type according to the usage scenario. All barcodes are on by default.

Turn on all barcode type *

Turn off all barcode type

Trun on / off all 1D barcodes type

Turn on all 1D barcodes *

Turn off all 1D barcodes

Trun on / off all 2D barcodes type

Turn on all 2D barcodes *

Turn off all 2D barcodes

Codabar

Enable Codabar

Disable Codabar

Codabar Start character/end character

Not sending Start character/end character *

Send Start character/end character

Codabar Length limit setting

Codabar Minimum length limit（0~50 bits）

Codabar Maximum length limit（0~50 bits）

Code 39

Enable Code 39

Disable Code 39

Code 39 check bit

Disable check bit *

Enable but not to send check bit

Send check bit

Code 39 Full ASCII

Turn on Full ASCII

Turn off Full ASCII *

Code 39 Length limit setting

Code 39 Minimum length limit（0~50 bits）

Code 39 Maximum length limit（0~50 bits）

Code 32  (code39 needs to be turned on)

Enable Code 32

Disable Code 32

Interleaved 2 of 5（ITF25）

Enable ITF25

Disable ITF25

Interleaved 2 of 5（ITF25）check bit

Disable check bit *

Enable but not to send check bit

Send check bit

Interleaved 2 of 5（ITF25）length selection

ITF25 random lengths（6-50 bits）*

ITF25  6 bits length

ITF25  8 bits length

ITF25  10 bits length

ITF25  12 bits length

ITF25  14 bits length

ITF25  16 bits length

ITF25  18 bits length

ITF25  20 bits length

ITF25  22 bits length

ITF25  24 bits length

Interleaved 2 of 5 Length limit setting

Interleaved 2 of 5 Minimum length limit（0~50 bits）

Interleaved 2 of 5 Maximum length limit（0~50 bits）

Industrial 2 of 5（Industrial 2 of 5）

Enable Industrial 2 of 5

Disable Industrial 2 of 5

Industrial 2 of 5 Length limit setting

Industrial 2 of 5 Minimum length limit（0~50 bits）

Industrial 2 of 5 Maximum length limit（0~50 bits）

Matrix 2 of 5（Matrix 2 of 5）（4-24 bits）

Enable Matrix 2 of 5

Disable Matrix 2 of 5

Matrix 2 of 5 Length limit setting

Matrix 2 of 5 Minimum length limit（0~50 bits）

Matrix 2 of 5 Maximum length limit（0~50 bits）

Code 93

Enable Code 93

Disable Code 93

Code 93 Length limit setting

Code 93 Minimum length limit（0~50 bits）

Code 93 Maximum length limit（0~50 bits）

Code 11

Enable Code 11

Diable Code 11 *

Code 11 Check bit output

Send check bit

Not to send check bit *

Code 11 check bit selection

Disable check bit *

Code 11   1 bit

Code 11   2 bits

Code 11 Length limit setting

Code 11 Minimum length limit（0~50 bits）

Code 11 Maximum length limit（0~50 bits）

Code 128

Enable Code 128

Disable Code 128

ISBT-128

Enable ISBT 128

Disable ISBT 128

GS1-128

Enable GS1-128

Disable GS1-128

GS1-128 Length limit setting

GS1-128 Minimum length limit（0~50 bits）

GS1-128 Maximum length limit（0~50 bits）

UPC-A

Enable UPC-A

Disable UPC-A

UPC-A check bit

Send check bit *

Not to send check bit

UPC-A convert to EAN-13

Enable UPC-A convert to EAN-13

Disable UPC-A convert to EAN-13 *

UPC-E

Enable UPC-E

Disable UPC-E

UPC-E check bit

Send check bit *

Not to send check bit

UPC-E extend to UPC-A

Enable UPC-E extend to UPC-A

Disable UPC-E extend to UPC-A *

EAN/JAN-8

Enable EAN/JAN-8

Disable EAN/JAN-8

EAN/JAN-13

Enable EAN/JAN-13

Disable EAN/JAN-13

UPC/EAN/JAN extracode

Ignore UPC/EAN/JAN extracode *

Decode UPC/EAN/JAN extracode

Adapt UPC/EAN/JAN extracode

EAN13 convert to ISBN

Enable EAN13 convert to ISBN code

Disable EAN13 convert to ISBN code *

EAN13 convert to ISSN

Enable EAN13 convert to ISSN code

Disable EAN13 convert to ISSN code *

GS1 DataBar（RSS14）

Enable GS1 DataBar

Disabel GS1 DataBar

GS1 DataBar Limited

Enable GS1 DataBar Limited

Disable GS1 DataBar Limited

GS1 DataBar Expanded

Enable GS1 DataBar Expanded

Disable GS1 DataBar Expanded

MSI code

Enable MSI code

Disable MSI code

MSI check bit output

Send MSI code check bit

Not to send MSI code check bit

MSI check bit selection

1 bit

2 bits

MSI code 2 bits check bit selection

MOD10/MOD10

MOD10/MOD11

MSI code length limit setting

MSI code minimum length

MSI code Maximum Length

Febraban（ Brazil bank document barcode）

Note: Please turn off the AIM ID function before enable the Frabraban function.

Febraban（ITF25）

Enable Febraban（ITF25）

Disable Febraban（ITF25）（Factory default）

Febraban（CODE128）

Enable Febraban（CODE128）

Febraban（CODE128）（Factory default）

Febraban parity

Enable Febraban parity

Disable Febraban parity（Factory default）

PDF417

Enable PDF417

Disable PDF417

Micro PDF417

Enable Micro PDF417

Disable Micro PDF417

QR Code

Enable QR

Disable QR

Micro QR

Enable Micro QR

Disable Micro QR

Data Matrix

Enable Data Matrix

Disable Data Matrix

Aztec Code

Enable Aztec

Disable Aztec

Appendix

Data code

0

1

2

3

4

5

6

7

8

9

A

B

C

D

E

F

Cancel current Settings

Cancel a serials of data you just set

Cancel a data you just set

save

Bar code type ID table

| Barcode type | HEX | CODE ID(default) |
| --- | --- | --- |
| All types | 99 |  |
| Codabar | 61 | a |
| Code128 | 6A | j |
| Code32 | 3C | < |
| Code93 | 69 | i |
| Code39 | 62 | b |
| Code11 | 48 | H |
| EAN-13 | 64 | d |
| EAN-8 | 64 | d |
| GS1 DataBar | 79 | y |
| GS1-128 (EAN-128) | 6A | j |
| 2 of 5 |  |  |
| Interleaved 2 of 5 | 65 | e |
| Matrix 2 of 5 | 76 | v |
| Industry 2 of 5 | 44 | D |
| UPC-A | 63 | c |
| UPC-E | 63 | c |
| ISBN | 42 | B |
| ISSN | 6E | n |
| Aztec Code | 7A | z |
| DataMatrix | 75 | u |
| PDF417 | 72 | r |
| Micro PDF417 | 53 | S |
| QR Code | 51 | Q |
| Micro QR Code | 51 | Q |

AIM  ID table

| Barcode type | AIM ID | Describe |
| --- | --- | --- |
| Codabar | ]Fm | m：0~1 |
| Code128 | ]C0 | m：0，1，2，4 |
| Code32 | ]A0 |  |
| Code93 | ]G0 |  |
| Code39 | ]Am | m：0，1，3，4，5，7 |
| Code11 | ]Hm | m：0，1，3，8，9 |
| EAN-13 / EAN-8 | ]Em | m：0，1，3，4 |
| GS1 DataBar | ]e0 |  |
| GS1-128 (EAN-128) | ]C1 |  |
| Interleaved 2 of 5 | ]Im | m：0，1，3 |
| Matrix 2 of 5 | ]X0 |  |
| Industry 2 of 5 | ]S0 |  |
| UPC-A/ UPC-E | ]Em | m：0，3 |
| ISBN | ]X0 |  |
| ISSN | ]X0 |  |
| Aztec Code | ]z0 |  |
| DataMatrix | ]dm | m: 0~6 |
| PDF417 / Micro PDF417 | ]Lm | m: 0~5 |
| QR Code / Micro QR Code | ]Qm | m: 0~6 |

ASCII table

|  | Decimalism |  |  | Hexadecimal |  |  | character |  | Decimalism |  |  | Hexadecimal |  | character |  | Decimalism |  |  | Hexadecimal |  | character |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 32 |  |  | 20 |  |  | <SPACE> |  | 64 |  |  | 40 |  | @ |  | 96 |  |  | 60 |  | ` |
|  | 33 |  |  | 21 |  | ! |  |  | 65 |  |  | 41 |  | A |  | 97 |  |  | 61 |  | a |
|  | 34 |  |  | 22 |  |  | “ |  | 66 |  |  | 42 |  | B |  | 98 |  |  | 62 |  | b |
|  | 35 |  |  | 23 |  | # |  |  | 67 |  |  | 43 |  | C |  | 99 |  |  | 63 |  | c |
|  | 36 |  |  | 24 |  | $ |  |  | 68 |  |  | 44 |  | D |  | 100 |  |  | 64 |  | d |
|  | 37 |  |  | 25 |  | % |  |  | 69 |  |  | 45 |  | E |  | 101 |  |  | 65 |  | e |
|  | 38 |  |  | 26 |  | & |  |  | 70 |  |  | 46 |  | F |  | 102 |  |  | 66 |  | f |
|  | 39 |  |  | 27 |  |  | ‘ |  | 71 |  |  | 47 |  | G |  | 103 |  |  | 67 |  | g |
|  | 40 |  |  | 28 |  | ( |  |  | 72 |  |  | 48 |  | H |  | 104 |  |  | 68 |  | h |
|  | 41 |  |  | 29 |  | ) |  |  | 73 |  |  | 49 |  | I |  | 105 |  |  | 69 |  | i |
|  | 42 |  |  | 2A |  | * |  |  | 74 |  |  | 4A |  | J |  | 106 |  |  | 6A |  | j |
|  | 43 |  |  | 2B |  | + |  |  | 75 |  |  | 4B |  | K |  | 107 |  |  | 6B |  | k |
|  | 44 |  |  | 2C |  | , |  |  | 76 |  |  | 4C |  | L |  | 108 |  |  | 6C |  | l |
|  | 45 |  |  | 2D |  | - |  |  | 77 |  |  | 4D |  | M |  | 109 |  |  | 6D |  | m |
|  | 46 |  |  | 2E |  | . |  |  | 78 |  |  | 4E |  | N |  | 110 |  |  | 6E |  | n |
|  | 47 |  |  | 2F |  | / |  |  | 79 |  |  | 4F |  | O |  | 111 |  |  | 6F |  | o |
|  | 48 |  |  | 30 |  | 0 |  |  | 80 |  |  | 50 |  | P |  | 112 |  |  | 70 |  | p |
|  | 49 |  |  | 31 |  | 1 |  |  | 81 |  |  | 51 |  | Q |  | 113 |  |  | 71 |  | q |
|  | 50 |  |  | 32 |  | 2 |  |  | 82 |  |  | 52 |  | R |  | 114 |  |  | 72 |  | r |
|  | 51 |  |  | 33 |  | 3 |  |  | 83 |  |  | 53 |  | S |  | 115 |  |  | 73 |  | s |
|  | 52 |  |  | 34 |  | 4 |  |  | 84 |  |  | 54 |  | T |  | 116 |  |  | 74 |  | s |
|  | 53 |  |  | 35 |  | 5 |  |  | 85 |  |  | 55 |  | U |  | 117 |  |  | 75 |  | u |
|  | 54 |  |  | 36 |  | 6 |  |  | 86 |  |  | 56 |  | V |  | 118 |  |  | 76 |  | v |
|  | 55 |  |  | 37 |  | 7 |  |  | 87 |  |  | 57 |  | W |  | 119 |  |  | 77 |  | w |
|  | 56 |  |  | 38 |  | 8 |  |  | 88 |  |  | 58 |  | X |  | 120 |  |  | 78 |  | x |
|  | 57 |  |  | 39 |  | 9 |  |  | 89 |  |  | 59 |  | Y |  | 121 |  |  | 79 |  | y |
|  | 58 |  |  | 3A |  | : |  |  | 90 |  |  | 5A |  | Z |  | 122 |  |  | 7A |  | z |
|  | 59 |  |  | 3B |  | ; |  |  | 91 |  |  | 5B |  | [ |  | 123 |  |  | 7B |  | { |
|  | 60 |  |  | 3C |  | < |  |  | 92 |  |  | 5C |  | \ |  | 124 |  |  | 7C |  | ¦ |
|  | 61 |  |  | 3D |  | = |  |  | 93 |  |  | 5D |  | ] |  | 125 |  |  | 7D |  | } |
|  | 62 |  |  | 3E |  | > |  |  | 94 |  |  | 5E |  | ^ |  | 126 |  |  | 7E |  | ~ |
|  | 63 |  |  | 3F |  | ? |  |  | 95 |  |  | 5F |  | _ |  |  |  |  |  |  |  |

Control character set (USB keyboard)

| Decimalism | hexadecimal | Corresponding keyboard value （Control character escape is turned off） | Corresponding keyboard value （Control character escape is turned on） |
| --- | --- | --- | --- |
| 0 | 00 | save | Ctrl+@ |
| 1 | 01 | Insert | Ctrl+A |
| 2 | 02 | Home | Ctrl+B |
| 3 | 03 | End | Ctrl+C |
| 4 | 04 | Delete | Ctrl+D |
| 5 | 05 | PageUp | Ctrl+E |
| 6 | 06 | PageDown | Ctrl+F |
| 7 | 07 | ESC | Ctrl+G |
| 8 | 08 | Backspace | Ctrl+H |
| 9 | 09 | Tab | Ctrl+I |
| 10 | 0A | Enter （Performance is affected by carriage return and line feed processing configuration） | Ctrl+J |
| 11 | 0B | Caps Lock | Ctrl+K |
| 12 | 0C | Print Screen | Ctrl+L |
| 13 | 0D | Enter （Performance is affected by carriage return and line feed processing configuration） | Ctrl+M |
| 14 | 0E | Scroll Lock | Ctrl+N |
| 15 | 0F | Pause/Break | Ctrl+O |
| 16 | 10 | F11 | Ctrl+P |
| 17 | 11 | Arrow ↑ | Ctrl+Q |
| 18 | 12 | Arrow ↓ | Ctrl+R |
| 19 | 13 | Arrow ← | Ctrl+S |
| 20 | 14 | Arrow → | Ctrl+T |
| 21 | 15 | F12 | Ctrl+U |
| 22 | 16 | F1 | Ctrl+V |
| 23 | 17 | F2 | Ctrl+W |
| 24 | 18 | F3 | Ctrl+X |
| 25 | 19 | F4 | Ctrl+Y |
| 26 | 1A | F5 | Ctrl+Z |
| 27 | 1B | F6 | Ctrl+[ |
| 28 | 1C | F7 | Ctrl+\ |
| 29 | 1D | F8 | Ctrl+] |
| 30 | 1E | F9 | Ctrl+^ |
| 31 | 1F | F10 | Ctrl+_ |

Control character set（serials port and USB-COM）

| Decimalism | hexadecimal | Corresponding keyboard value |
| --- | --- | --- |
| 0 | 00 | NUL |
| 1 | 01 | SOH |
| 2 | 02 | STX |
| 3 | 03 | ETX |
| 4 | 04 | EOT |
| 5 | 05 | ENQ |
| 6 | 06 | ACK |
| 7 | 07 | BEL |
| 8 | 08 | BS |
| 9 | 09 | HT |
| 10 | 0A | LF |
| 11 | 0B | VT |
| 12 | 0C | FF |
| 13 | 0D | CR |
| 14 | 0E | SO |
| 15 | 0F | SI |
| 16 | 10 | DLE |
| 17 | 11 | DC1 |
| 18 | 12 | DC2 |
| 19 | 13 | DC3 |
| 20 | 14 | DC4 |
| 21 | 15 | NAK |
| 22 | 16 | SYN |
| 23 | 17 | ETB |
| 24 | 18 | CAN |
| 25 | 19 | EM |
| 26 | 1A | SUB |
| 27 | 1B | ESC |
| 28 | 1C | FS |
| 29 | 1D | GS |
| 30 | 1E | RS |
| 31 | 1F | US |

Some function configuration instructions and examples

Custom suffix configuration example

Configure barcode prefix / suffix by scanning, each prefix or suffix can be up to 10 characters.（To ensure that custom suffixes can be output, configure the scanner's custom suffix / suffix output option to be turned on.）

Example 1.1: Add custom prefix XYZ to all types of barcodes

Query appendix barcode type ID table, HEX value of all barcodes is 99. Query the visible character ASCII table. The HEX value corresponding to XYZ is 58,59,5A.

Scan the configuration code custom prefix, the bar code scanner will issue two beeps, and then scan the appendix data and edit the 9, 9, 5, 8, 5, 9, 5, A in the barcode, Save to complete the configuration.

If you need to modify the scanned bar code before saving, you can also scan to cancel the previous data or cancel the previous data and reconfigure. If you need to abandon the configuration midway, scan directly to cancel the current settings.

Example 1.2: Add custom prefix R to QR code

Query the appendix barcode type ID table. The HEX value of the QR code is 51. Query the visible character ASCII table. The HEX value corresponding to R is 52.

Scan the configuration code to customize the prefix, then scan the data in the appendix and edit 5,1,5,2 in the barcode, save, and complete the configuration.

Example 1.3: Cancel custom prefix of QR code

When custom suffixes are used, no additional characters are saved after the barcode type characters to save the custom suffixes for this type of barcode.

Scan the configuration code to customize the prefix, then scan the appendix data and edit 5, 1 in the barcode, save, and complete the configuration.

Note: If there is a prefix added for all barcodes before, the QR code prefix will be restored to the prefix added for all barcodes after configuration.

If you need to clear the prefix / suffix added for various barcode types, scan to clear all custom prefixes and clear all custom suffix configuration codes.

Barcode Length Limit Configuration Example

When configuring the minimum barcode length configuration, you must ensure that the configured minimum length is not greater than the current maximum length configuration, otherwise an error will be prompted. Similarly, when configuring the maximum bar code length, you must ensure that the configured maximum length is not less than the current minimum length configuration.

Example 2.1: Configure Code 128 barcode length to 4-12 digits

Scan configuration code Code 128 minimum length limit, then scan 4 in the appendix data and edit the barcode, save,

Scan the configuration code Code 128 maximum length limit, then scan the appendix data and edit the 1, 2 in the barcode, save, and complete the configuration.

Example 2.2: Configure the length of Interleaved 2 of 5 barcode to 14 digits

Configure Interleaved 2 of 5 barcode length 14 bits can directly scan and configure the barcode ITF25 14-bit length for configuration, or configure the maximum and minimum barcode length:

Scan configuration code Interleaved 2 of 5 minimum length limit, then scan appendix data and edit 1, 4 in barcode, save,

Scan the configuration code Interleaved 2 of 5 maximum length limit, then scan the appended data and edit the 1, 4 in the barcode, save, and complete the configuration.

Example 2.3: Configure Code 39 barcode length to any supported length

Scan configuration code Code 39 minimum length limit, then scan appendix data and edit 0 in barcode, save,

Scan the configuration code Code 39 maximum length limit, then scan the appendix data and edit the 0 in the barcode, save, and complete the configuration.

USB keyboard send speed configuration example

f the client PC has weak performance and is prone to transmission error conditions, you need to customize the USB keyboard sending speed to a slower speed, such as 50ms:

Scan the configuration code to customize the sending speed, then scan the appended data and edit the 5,0 in the barcode, save it, and complete the configuration.

Warning sound

When the data transmission is abnormal, the scanner will issue four continuous alarm sounds. If this happens, please check whether the connection line is normal.

Reading tips

In order to obtain a good reading effect, the aiming beam emitted by the scanning gun should be aimed at the center of the barcode, but it can be aimed in any direction for reading.

Hold the scanner in front of the barcode and press the button to align the collimator beam with the center of the barcode.

The closer the scanning gun is to the barcode, the smaller the aiming beam; the further away the scanning gun is from the barcode, the larger the aiming beam. If the bar code is small, the scanning gun should be close to the bar code; if the bar code is large, the scanning gun should be a little far away from the bar code, so that it is easier to read the bar code correctly.

If the barcode is highly reflective (for example: coated surface), you may need to tilt the scanner at an angle to successfully scan the barcode.

Safety

When using the scanning gun, the illumination light is strong. Do not look directly or aim at your eyes to avoid discomfort or injury.