诶

Refrigerator

Service Manual

Models:  HD-273FN(R2255W)

Features

●    Energy efficiency

●    Micro foam technology

●    Low noise operation

●    Thick insulation for energy efficiency

●    Latest No. 2 micro-biological technology

0

CONTENT

CONTENT

1.  Content.

1

2.  Product  code  illumination  and  Series  Introduction   2

3.  Specifications....      3

4.  Safety  precautions....    4

5.  Function  Schedule........    7

6.  System  Flow  Chart.....    8

7.  Circuit  Diagram.....       9

8.  Maintenance  Service  and  Trouble  Shooting....    10

1

3

Safety precautions

Safety precautions

4

Safety precautions

5

Safety precautions

6

Function  Schedule

Function Schedule

●    Energy efficiency

●    Micro foam technology

●    Low noise operation

●    Thick insulation for energy efficiency

7

System Flow Chart

System Flow Chart

1 Schematic diagram of System Flow Chart

Flows in the sequence: 1->2->3->4->5>1

8

Circuit Diagram

Circuit Diagram

1 Schematic Circuit Diagram

9

Maintenance Service and Trouble Shooting

Maintenance Service and Trouble Shooting

1. No cooling

No cooling

If compressor

can run after it stop to

run five minutes and

give power.

NO

Check and repair according to

the section of "the compessor

doesn't work".

YES

If compressor

can run naturally and

temperature raising

of condenser is

very low

YES

NO

Refrigerant

leak,check and

repair charge

refrigerant again.

Block by ice or dirty

things,change filter and change

refrigerant again.

!

When charging refrigerant,we must

notice to make refrigerant true.

10

Maintenance Service and Trouble Shooting

2 The compressor doesn't stop

Compressor doesn't

stop

Does the refrigerator

refrgerate?

NO

Check and repair according to

the section of "No cooling".

YES

Is  cooling capacity

insufficient.

NO

Check and repair according to

the section of "Cooling capacity

is insuffcient".

YES

1 The thermostat  was

set at the coldest position.

2 The ambient temperature is very high or

the refrigerator is near by heat resource.

3 The door was opened too many

times or there are more

fresh food inside.

NO

Adjust and instruct

consumer how to use.

NO

Check and repair the main

control panel

11

Maintenance Service and Trouble Shooting

3 Cooling capacity is insufficient

Cooling capacity is

insufficient

Is the ambient YES

temperature low than

16°C

Check the therostat is

correct?

YES

NO

Does the refrigerant

leak siowly?.

Adjust and instruct

consumer how to use.

Repair

12

Maintenance Service and Trouble Shooting

4  The compressor doesn't work

The compressor doesn't

work.

Does the refrigerated

room lamp illuminate

when the door is

opened?

NO

Check the power

source and power

cord?

NO

Repair

YES

YES

Is the ambient

temperature low than

16

YES

Check the therostat is

correct?

Adjust and instruct

consumer how to use.

Does the control

panel has 220V power

output?

NO

Check and repair the

control panel.

YES

Does the compressor

has 220V power

output?

NO

Check all cords and

power cords.

YES

Check and repair

compressor and accessory.

13

Maintenance Service and Trouble Shooting

5  Normal Phenomena—Not Troubles for Refrigerators

In a domestic refrigerator, there is not only a complicated electric control

system, but also a refrigerating system that is difficult to discern whether it

is in a good working condition. Both the systems are related and affect

each other. If a certain part of a refrigerator works abnormally .Its

refrigerating efficiency will decrease ,operation properties will become

unstable ,or even it cannot be used normally for those serious cases.

Refrigerators are generally of larger volume ,once troubles appear ,to

send them to a service department is really a tough thing ,if such is the

case ,the user will always be in suspense ,sometimes ,normal phenomena

will be erroneously regarded as troubles .Therefore ,before we deal with

the topics of frequently occurring troubles of refrigerators and their

remedies ,we should firstly give a brief account of some normal

phenomena which are not troubles .In case any one of such phenomena

occurs ,there is no need to worry about it ,and the user can use it at total

ease .

1).When the compressor of a refrigerator has just stopped running ,a

rumbling sound can be heard from inside its evaporator .This is a sound

caused by the flowing of refrigerant in the evaporator tubing .Because the

pressure difference is still greater after the compressor has just stopped

running ,the refrigerant will flow for a certain time , therefore ,this sound is a

normal phenomenon.

2).A click sound can often be heard from the refrigerator .This is a

normal sound produced by the pull-in or release of the armature of a

current deadweight start relay when starting the compressor .The

compressor motor will produce a slight and uniform sound while it is

running .This sound is not easy to be heard in the daytime ,but of course it

can be heard distinctly at night.

3).The compressor consists of an electric motor and a compressing

apparatus .During its normal operatio ,the motor's stator core and

windings will rise to a temperature in the range of 100°C~110°C ,and the

temperature of the piston and cylinder of the compressing mechanism can

also reach as above 100°C due to the heat produced when compressing

refrigerant .Most of the heat radiates to the air through the compressor

casing ,therefore ,its casing is generally at a temperature between 85°C ~90°C

,it is very hot ,particularly in summer when the ambient temperature is

higher .All these are normal phenomena .

4).For the direct cooling refrigerator ,a kind of irregular crack sound can

be often heard when the compressor is running for a certain period of time

or has just stopped its running .This sound is caused by the stress relief

due to expansion and contraction when temperature changes ,and will not

affect the normal application of refrigerator .

14

Maintenance Service and Trouble Shooting

6 Common Troubles in Refrigerators and Their Remedies

Causes for troubles occurring in refrigerators are closely related to the

quality of components and workmanship in assembling by manufacturers

as whether refrigerators are properly used and maintained .The parameters

generally used to express the working conditions of a refrigerator include

the temperature inside the refrigerator ,operation rate ,electric power

consumption ,noise level ,and other functional indexes .If any one of these

parameters is beyond its permissible range ,this indicates that there is a

fault or trouble in the refrigerator .

During the whole service life of a refrigerator , the probability of troubles

occurring within a union time is called its failure rate .Making a comparison

between  the control circuit system of a refrigerator and its refrigerating

system ,we can find that the failure rate of the former is higher ,and that of

the thermostat is the highest . In troubleshooting , the first thing you must do

is to determine where the trouble comes from --- the control system or the

refrigerating system .There is general no trouble indicating instrument

mounted on the dome stic refrigerator ,locations and natures of troubles

should be determined according to their respective

features ,therefore ,experience in servicing is very important to

troubleshooting. Service technicians with rich experience can correctly

locate them and take reasonable remedy measures based on their

comprehensive analysis of trouble characteristics as well as operating

conditions for various kinds of refrigerators .

Three Essentials for Checkup

1) Look

a)   Check the tubing of refrigerating system for cracks and various

welding points for leaks ;if leakage occurs ,an oil stain can be seen

definitely .

b)   Check the suction and exhaust pressure values (high pressure and

low pressure) of compressor to see whether they are normal .

c)   Check the conditions of frost attached to its evaporator and gas

return tube .It is abnormal if frost has formed on part of the evaporator or

there is no frost attached to it .

d)   Pay attention to the speed of temperature drop inside freezer

compartment .It is abnormal if the speed of temperature drop is obviously

slower than the corresponding normal speed .

e)   Check the environment to see whether it is suitable for placing a

refrigerator .

f)

layer .

Check refrigerator door seal ,case ,table surface and heat insulation

g)   Look at the main control board to ascertain if various indication

states are normal.

2) Listen

a) Listen to the noises produced when the compressor is running

15

Maintenance Service and Trouble Shooting

Hums from a fully enclosed aggregate unit is the sound caused by

overload indicating that the motor cannot be started normally ,meanwhile ,a

clattering sound can be heard from inside the start relay ,which is produced

because the start contacts cannot be released normally .A whistling sound

is caused by the high pressure gas flowing out of the crack of the pressure

tube inside the compressor ,and clucks are the sound of striking after the

suspended spring inside the compressor has broken .

During the normal operation of compressor, a slight and uniform hum

sound due to undulation of electric current can be heard generally ,this is a

normal phenomenon .However ,if it sounds like "tong ,tong ",i.e., an

impact sound inside the compressor ,this means that a large quantity of wet

vapour of refrigerant or refrigerating oil has come into the compressor

cylinder ;if it sounds like "dang ,dang", a striking sound of metal parts

inside the compressor ,this means that some moving parts have loosened

(note to differentiate this sound from those formed during starting or

stopping the compressor).

b) Listen to the sound caused by the flowing of gas in the evaporator

Open the refrigerator door while the compressor is in operation, incline

your ear and listen attentively the gas flow sound inside the evaporator .If it

sounds like gentle whistling accompanied by a sound similar to water

flowing ,this is the sound produced by the normal circulation of refrigerant

within the evaporator .In case only the gas flowing sound can be heard and

there is no water flowing sound ,this indicates that the refrigerant has

already percolated .If neither the flowing sound nor the gas sounds from

the evaporator can be heard ,this means that the filter or capillary has been

clogged .

3)Touch and Feel

a)   Feel the compressor when running ,its temperature should be

generally less than 90°C in the normal state (it may exceed 90°C in case

of running for a longer period of time ).

b)   After the compressor has operated normally for 5~10 minutes  ,

ouch and feel the condenser ,the temperature of its upper part should be

higher than that of its lower part (or its right part is hotter than its left

part ,depending on the type of condenser coil ),this indicates that the

refrigerant is circulating . If the condenser is not hot ,this means the

leakage of refrigerant .In case the condenser radiates heat for only several

minutes and then cools down ,this means that the filter and capillary have

been clogged .As for the forced air cooling condenser ,hot air will be blown

out of it ; this means that the system is out of order .

c)   Feel the filter's temperature .During the normal operation of

refrigerating system ,the temperature on the filter's surface should be a

little higher than the ambient temperature ;if you touch it with your

hand ,you will have a sense of slight heat .In case dew condensation

appears due to the fact that its temperature is obviously lower than the

16

Maintenance Service and Trouble Shooting

ambient temperature ,this means that most meshes of its screen has been

clogged ,resulting in an obstructed flowing of refrigerant ,thus causing a

drop in temperature due to throttling .

d)  Feel the temperature of exhaust gas from the refrigerating

system .the exhaust gas should be very hot and this is the normal working

state .For those refrigerator with enclosed type of compressor refrigerating

system, no frost or dew drop will from on the gas suction

tube ,otherwise ,there is something wrong in the system (Frosting and dew

condensation may appear for a very short time period when just starting the

machine ,this is a normal phenomenon ).

Because a refrigerator is a combination of several components ,they are

related and have influence on each other In case an abnormal

phenomenon has been found through the above-mentioned checkups ,you

need not to make a hasty conclusion based on only one abnormal

phenomenon .It is advisable to find out two or more abnormal

phenomenon ,or conduct troubleshooting comprehensively with the aid of

instruments or other ways ,because several kinds of troubles may share a

common abnormal phenomenon ,and two or more abnormal phenomena

may occur simultaneously due to a certain trouble .With this method ,you

can reject some suspicious troubles and finally make a correct judgment .

7 Analysis of Troubles and Troubleshooting

A.   Poor Refrigerating Effect

The so-called "poor refrigerating effect " refers to the fact that the

refrigerator can operate and refrigerator normally ,but the temperature in

the refrigerator cannot drop to the prescribed value under the stipulated

working conditions .In view of there are many causes for this

phenomenon ,we are going to discuss and analyze it in the following 7

aspects :

1)   Leakage of refrigerant

Analysis of Trouble

The leakage of refrigerant in the system will result in an insufficient

refrigerating capacity ,the resulting phenomena are its lower gas suction

pressure and exhaust pressure as well as higher exhaust gas

temperature. The exhaust tube feels rather hot ,and a continuous gas

flowing sound louder than usual can be heard at the outlet of the

capillary ,and no frost or a smaller quantity of loose frost appears on the

evaporator .After shut down ,the balance pressure in the system is usually

lower than the saturation pressure corresponding to the same ambient

temperature .

Remedy

In case there is leakage of refrigerant from the system ,do not hurry to

recharge it with refrigerant ,manage to find out leak points immediately ,and

make a note of from where it leaks ---welding points and parts .After having

them repaired ,recharge refrigerant .

17

Maintenance Service and Trouble Shooting

many joints and sealed surfaces in a

There

are

so

refrigerator ,accordingly ,quite a lot potential leak points do exist in the

system .In troubleshooting ,pay attention to those parts that are liable to

leak ,check main connection points for oil seepage and tubing for cracks .If

there is no severer leak point ,charge the system with nitrogen and detect

leak points with the commonly adopted method ,repair

them ,evacuate ,charge the system with refrigerant ,and then turn on the

refrigerator to make a test run .

2)   Too much refrigerant charged into the system

Analysis of Trouble

a)   In case the amount of refrigerant charged into the system exceeds

its nominal capacity ,the superfluous refrigerant will of course take some

space of the evaporator ,thus reducing its heat-dissipation area and hence

the refrigerating efficiency of the system .The abnormal phenomena

caused by this reason are as follows :the gas suction and exhaust

pressures are generally higher than their respective normal values ,the

temperature of its condenser is higher and the electric current of the

compressor rises ,loose frost forms on the evaporator ,the refrigerator

temperature drops slowly ,and frost appears on the gas return tube .

b)   In case excessive refrigerant has been charged, the liquid

refrigerant that cannot evaporates in the evaporator will return to the

compressor ,and thus a phenomenon of "liquid striking" will occur .The

liquid refrigerant evaporates and effervesces as soon as it flows into the

refrigerating oil at the bottom of compressor .If the condition is

severe ,foams will fill all over inside the compressor housing and be sucked

by the piston ,causing damage to the components of compressor .

Remedy

According to the operating procedures, it is mandatory to turn the

achine off ,and several minutes later ,open the refrigerant charging tube

and et the refrigerant escape from it ,replace the dry filter ,recharge

refrigerant after evacuating ,and then seal the charging port .

3)   There is air left in the refrigerating system

Analysis of Trouble

Residual air in the refrigerating system will reduce its refrigerating

efficiency .The prominent phenomena are the increase in its gas suction

and exhaust pressures (however ,the gas exhaust pressure will not exceed

its rated value ),obvious increase in temperature in the segment from the

compressor outlet to the condenser inlet .Because there is air in the

system ,both the gas exhaust pressure and temperature will rise ,and

moreover ,the gas flowing sound is intermittent and obviously louder .

Remedy

After shut down for several minutes ,open the tubing ,evacuate it and

then recharge the system with refrigerant .

4)   Low efficiency of compressor

18

Maintenance Service and Trouble Shooting

Analysis of Trouble

Low efficiency of a refrigerating compressor refers to the actual

reduction of gas discharge capacity ,and hence the corresponding

reduction of refrigerating capacity ,under the condition that the refrigerant in

the system is unchanged .This phenomenon occurs mostly when the

compressor has been used for quite a long time ,its moving parts have

worn to a considerable degree ,fit clearance between various parts have

increased and sealing property of its gas valve has deteriorated ,resulting

in an decrease in its actual gas discharge capacity .

Methods of Judgment

Measure the high and low pressures with pressure gauges to see

whether they are normal .If an abnormal sound comes from the compressor

or the temperature of its causing is too high ,Cut the discharge port open

and operate the compressor ,feel it if there is pressure at the discharge port

with your finger (For a normal compressor ,its discharge port is difficult to

be blocked with your finger if you just apply a little effort).

5)   Too thick frost layer formed on evaporator

Analysis of Trouble

If a direct cooling refrigerator is to be used for a prolonged period of

time ,be sure to defrost its evaporator regularly ;if you fail to do so ,the frost

layer on the evaporator tubing will become thicker and thicker .Heat

conduction will be severely affected when the whole tubing is wrapped with

a transparent ice layer ,and finally the temperature in the refrigerator

cannot drop down to the prescribed range .

Remedy

Turn the refrigerator off , and make preparations for defrosting ,open the

refrigerator door to let the air come in ,or use a fan to speed up air flowing

so as to shorten the time needed for defrosting .Never strike the frost layer

with any iron tools or wood stick in order to avoid damage to the evaporator

tubing .

6)   Refrigerating oil left in evaporator tubing

Analysis of Trouble

During the process of refrigerating cycles , a little refrigerating oil may

remain in the evaporator tubing ,and after a longer time of operation ,if

there is considerable refrigerating oil left in the evaporator ,the heat

conduction effect will be severely affected ,thus causing a poor refrigerating

effect .

Remedy

It is quite difficult to determine whether this trouble is caused by the

refrigerating oil left in the evaporator tubing ,because this kind of

phenomenon and other several trouble are tends to be confused .Generally

speaking ,you can make a judgment based on the frost formed on the

evaporator .If the frost does not cover the evaporator , and moreover ,it is

loosely formed ,you can make a judgment that the deterioration of

19

Maintenance Service and Trouble Shooting

refrigerating effect is due to the accumulation of refrigerating oil left in the

evaporator tubing in case no other troubles have been found . To clear off

refrigerating oil left in the evaporator ,dismantle the evaporator ,purge it

thoroughly and then dry it .In case it is difficult  to  dismantle  it  ,charge

refrigerant from the evaporator inlet to wash it several times ,then purge

and dry it with nitrogen .

7)   Flowing obstructed in refrigerating system

Analysis of Trouble

Because the refrigerating system was originally not purged

thoroughly ,some of the filter screen meshes have been clogged by dirt

accumulated in the filter after using it for a certain time of period ,thus

resulting in a decrease in flow rate ,and hence a poorer refrigerating effect .

The abnormal phenomena caused by this kind of slight clogging in the

system are as follows: the gas discharge pressure is lower ,the

temperature of discharged gas drops down ,the position clogged has a

temperature lower than that in normal conditions ,and for a severe

clogging ,even dew condensation as frosting may appear .

Remedy

Purge the tubing , and after replacing the dry filter with a new one or

clearing it thoroughly ,recharge the system with refrigerant and seal the

charging port .

B.    No Refrigerating

The phenomenon that the compressor runs normally , but no frost (or

only a little frost ) appears on the evaporator ,and the refrigerator

temperature does not drop down is called "no refrigerating". There are

many causes for this trouble and it is relatively complicated , too . In

servicing ,special attention to finding direct causes for this

phenomenon .Three main potential causes for this trouble are analyzed

below :

1)  All refrigerant in the system has leaked out

Analysis of Trouble

Leak points in the refrigerating system have not been found and

repaired timely ,thus resulting in the total escape of refrigerant .There are

two kinds of leakage :I) slow leakage for instance ,when we want to use a

refrigerator that has been put out of quite a long time ,we find the leakage

of refrigerant ,or in the course of its operation ,we have found that the

refrigerator is gradually becoming not so cold and finally no refrigerating

cab be achieved ; ii)  fast leakage in that case ,all the refrigerant will

escape swiftly due to abrupt rupture of system tubing .

Symptoms of total leakage of refrigerant are mainly asfollows : the

compressor can be started easily (if there is no damage to the compressor

parts) and its operating current becomes low ,its gas suction pressure is

high and discharge pressure is lower ,the gas discharge tube feels rather

cold ,no sound of gas eruption from the liquid in the evaporator can be

20

Maintenance Service and Trouble Shooting

heard ,and no will gush out of the process tube if you cut it open after

shutdown .

Remedy

Check the whole machine , particularly those locations which are liable

to leak .After leak points have been found ,repair them or replace them with

new parts according to specific circumstances ,then evacuate the system

and finally charge refrigerant.

2)  Refrigerating system has been clogged

a)  Clogged with Ice

Analysis of Trouble

The dryness treatment for the major parts in the refrigerating system

has been performed improperly, the effect of air purging for the whole

system is not good ,or the water content in the refrigerant is out of

standard ,all these will cause the expansion valve to be clogged with

ice .The symptom for this trouble are as follows :the refrigerator sometimes

can refrigerate and sometimes cannot :the refrigerator compartment works

normally at the beginning ,but after working for a certain period of

time ,frosting begins at the clogged position ,evaporation temperature

reaches below 0°C , water will accumulate at the narrow part of the capillary

and clog it gradually ,then frost begins to melt at the evaporator ,no gas

flowing sound can be heard ,and the gas suction pressure assumes a state

of vacuum .Note that these phenomena will appear intermittently ;

sometimes ,the refrigerator works well ,and sometimes not well .In order to

determine whether it is clogged with ice ,heat the suspicious points with hot

water to make ice melt ,and after a while if a gas flowing sound resulting

from an abrupt gush is heard and the gas suction pressure rises as well ,it

can be confirmed that the trouble is caused by ice clogging .

Remedy

If there is too much moisture in the refrigerating system, it is advisable

to release the refrigerant, purge the tubing with nitrogen, and then charge

the system with the filtered refrigerant. However, the commonly adopted

method is to connect a filter with moisture absorbers (such as silica-gel,

anhydrous calcium chloride) to the refrigerating system so as to filter out

the moisture from the system, then replace the filter, evacuate it again and

finally charge the system with refrigerant.

b)  Clogged with dirt in capillary

Analysis of Trouble

The capillary inlet is a place where coarse-grained dirt or refrigerant oil

in the system will tend to settle down and clog it. In case considerable dirt

accumulates there, the whole filter screen may be blocked totally, making

the refrigerant unable to go through. Clogging with dirt exhibits the same

symptoms as that with ice, namely, higher gas suction pressure, lower

temperature of discharged gas, and no gas flowing sound from the

evaporator. The difference between the both is as follows: if the clogging is

21

Maintenance Service and Trouble Shooting

caused by dirt , tapping the suspicious clogged point (generally in the

capillary or the connection joint of the filter) may cause the refrigerant to

flow through partially, resulting in some corresponding change, whereas in

that case if you heat it with a hot towel, no reaction will happen, no flowing

sound can be heard, moreover, there is no periodic change. After rejecting

the possibility of ice clogging, it can be generally considered as the

clogging due to dirt .

Remedy

Dismantle the system, remove the dry filter from it, purge the tubing with

nitrogen, install a new filter, then evacuate it and finally charge it with

refrigerant.

C) Clogging in filter

Analysis of Trouble

Total clogging of a filter is rarely to occur. This trouble is mostly caused

by the paste-like matter formed from the substances filled into the system

or other dust after the refrigerator has been used for a longer time, or by

the dirt accumulated gradually inside the filter. Sometimes, tapping the filter

nay cause a passage for flowing, Touching it with your hand, you will feel

that it is cooler compared with its temperature in the normal state.

Remedy

The same as described in the capillary clogging with dirt.

3) Troubles of Compressor

a)  Breakage of gas suction and discharge valve blocks

Analysis of Trouble

The compressor works by means of the opening and closing of gas

suction valve and discharge valve to suck and discharge the refrigerant. If

the valve block is broken, the refrigerant can not be discharged, and hence

no refrigerating can be achieved.

Method for making judgement

It is quite difficult to differentiate this trouble from others because they

often have similar symptoms. In repairing, firstly, hear attentively if there is

some abnormal sound coming from the compressor (sometimes , the

broken pieces of valve block may strike against the cylinder), and feel the

compressor casing with your hand to ascertain whether it is too hot, this is

also helpful to the troubleshooting; secondly, measure the pressures at the

high and low pressure ports of the compressor with pressure gauges, if the

gas suction valve block is broken, the suction pressure gauge pointer will

swing violently and the suction pressure is very high, whereas when the

gas discharge valve block is broken , the discharge pressure gauge pointer

will swing drastically and the discharge pressure is very high. In the both

cases, stop the compressor at once, and if technique is available, open the

cylinder cover and check up the valve block, repair it, or replace it with a

new one,

C. Sudden stop of Compressor during Its Running

22

Maintenance Service and Trouble Shooting

The cause of sudden stop of a compressor during its running is mostly

that the gas suction pressure and/or discharge pressure exceed their

respective prescribed ranges, thus making a pressure-operated protective

relay shut off the power to the compressor and stop it. In the following we

discuss mainly the reasons for causing excessively high gas discharge

pressure and low suction pressure.

1)  Stoppage due to excessively high gas discharge pressure

A)  Too much refrigerant charged into the system

Analysis of Trouble

The phenomenon, such as loose frosting and poor refrigerating effect,

may occur if excessive refrigerant has been charged into the system.

Superfluous refrigerant will occupy a certain space of the evaporator, thus

reduce its heat dissipating area, and the phenomenon of "liquid striking"

may occur, too. Meanwhile, dew or frost condensation may occur on the

gas return tube, and the gas discharge pressure will obviously rise, when it

reaches the threshold value, the protective relay will actuate and shut off

the power supply to the compressor.

Remedy

Open the tubing, re-evacuate and then charge the system with a proper

quantity of refrigerant.

b)  Air left in the system

Analysis of Trouble

The residual air in the system will circulate together with the refrigerant

in the system. The major symptoms caused by this residual air is higher

gas discharge pressure, higher discharged gas temperature (the gas

discharge tubing is considerably hot when you feel it with your hand), and

poorer refrigerating effect. Furthermore, the gas discharge pressure will

exceed its normal value when the compressor has run for a period not too

long, thus making the protective relay actuate and bring to a stoppage.

Remedy

Check up how the air has been left in the refrigerating system.

Generally, there are two possibilities: one is that the air has been sucked

into the system when repairing due to carelessness, or it has not been

purged out totally when evacuating the system; the other is that there are

leak points at the low pressure end of the refrigerating system. Leak points

appear mostly in those low temperature parts or assemblies, because the

evaporation temperature is lower for such low temperature devices,

especially at the low pressure end, it is easier for the air goes into the

system. Once it has been ascertained that air does exist in the system, you

have to open the tubing, re-evacuate it and then charge it with refrigerant.

2)  Stoppage due to electric troubles

a)  Thermostat is out of control

Analysis of Trouble

In case the thermostat does not work in its good order or its

23

Maintenance Service and Trouble Shooting

temperature sensor has not been installed properly, frequent stoppage is

also likely to occur.

Remedy

Try to adjust the temperature sensor's position until the compressor can

be started and stopped normally. If this cannot be achieved, and the

stoppage still occurs repeatedly, it is most likely that the mechanical parts

or contacts are out of order, disassemble the thermostat, make a thorough

checkup and repair it.

b)  Overload of electric motor

Analysis of Trouble

Probably, too many things have been put into the refrigerator, and

hence the thermal load exceeds its refrigerating capacity ; or in case the

power supply voltage drops considerably, the current flowing through the

motor will increase drastically making the thermal protector actuate and the

fuse blown , and hence the motor stops running. If the motor is still running

continuously in such a case , its windings will be burned out .

Remedy

Reduce the thermal load, pay attention to the variation in voltage of the

power supply.

c)  Abnormal thermal protection

Analysis of Trouble

The compressor current is within its normal range, but the thermal

protector actuates repeatedly.

Remedy

Replace the thermal protector with a new one.

3) Sudden stoppage due to other causes

Normal Stoppage

Start and stop of the compressor is generally controlled by a thermostat .

When the temperature in the refrigerator reaches its desired value, the

thermostat will shut down the compressor automatically. Never take this

normal operation as a trouble, care should be taken to differentiate it from

other real troubles in servicing.

D. Compressor won 't start

In case the compressor cannot be started, you must find out the origins

of this trouble through checkup step by step, because probably there are

many causes, including those electrical and mechanical.

1)  Inspect the power supply to see whether it is connected to the

compressor circuit.

Analysis of Trouble

In case the compressor cannot be started, this will generally exhibit in

the power supply circuit, for instance, power failure, poor contact of switch,

and blown fuse. Make a comprehensive analysis of these phenomena, fine

out its real cause and take correct measures to remove this trouble.

Remedy

24

Maintenance Service and Trouble Shooting

a) Check the input power circuit to see whether where is voltage of the

power supply,   namely, the circuit which is connected to the knife switch.

This can be determined with an avometer or a test pencil. If a blown fuse is

found, ascertain and remove its cause, then replace it with a new one of

the same specifications.

b)  Check the compressor accessories, including its thermal protector

and relay. In case the thermal protector is damaged, the compressor

cannot be powered on. If the relay is out of order, the motor will not run and

hum sound can be heard from it after the compressor is turned on, in that

case, shut it down immediately, otherwise, the motor windings will be

burned out in case this condition lasts longer.

c)  Check the relay contacts and plugs to see if they are perfect and

work reliably. Poor contact may cause the motor not running or humming.

2) Check the circuit voltage to see whether it is normal.

Analysis of trouble

If the circuit voltage is obviously lower than its rated value, it will be

difficult to start the motor, and a hum sound can be heard from it.

Remedy

Measure the voltage with a voltmeter, if it is really too low, give

directions to the user for buying a stabilizer so as to step up the voltage,

thus normal operation can be achieved.

3) Check the thermal relay to see whether its contacts are closed.

Analysis of Trouble

The contacts of thermal relay sometimes may be open due to the

leakage of temperature sensing agent from the temperature sensor.

Remedy

Remove the relay cover to check up its contacts, if they are open, this

means that the original setting is not properly set or temperature sensing

agent has leaked out of the temperature sensor. Try to turn the adjusting

stem of this value in the direction of the lower temperature graduation, then

check the contacts to see whether they are closed. If they are still not

closed, dismantle the temperature sensing disc and then immerse it into

warm water to see whether the contacts actuate, if not, it can be

preliminarily determine that temperature sensing agent has leaked out, and

it must be replaced with a new thermostat.

4) Motor troubles and other electric faults

a) Motor windings have been burned or short-circuited between turns

Analysis of Trouble

When motor windings have been burned or short-circuited between

turns, the fuse will be blown repeatedly, and the blowout occurs particularly

at the instant when you close the knife switch.

Remedy

Check the terminals and the outer casing to see whether they are

short-circuited, and measure the resistance of each phase with an

25

Maintenance Service and Trouble Shooting

avometer. If short circuit occurs or the resistance of a certain phase is low,

this means that short circuit does exist in the windings and/or between

turns, and insulation layers have been burned or deteriorated. A

megameter can be used in this inspection, too. I f the insulation resistance

is lower than 2 M, this means that the insulation layer is already

breakdown. If the motor has been burned, repair it or replaced it with a new

one.

b) Fault of control relay

Analysis of Trouble

Overheat, burnout or wear of control relay contacts may occur generally.

All these will cause poor contact in electricity.

Remedy

Dismantle it to repair, or replace it with a new one.

c)  Poor electric contactor in thermostat

Analysis of Trouble

Burnout of contactor and leakage of temperature sensing agent may

occur generally.

Remedy

Replace the old with a new one.

d)  Check the terminals for loose connection and electric circuits for

other abnormal phenomena.

5) Mechanical faults of compressor

a)  Seizing of shaft

This phenomenon is caused mostly by poor lubrication, such ad

insufficient quantity of lubricant, clogging in the lubricant oil line, or

intermittent lubricant oil supply. Dirt and other impurities in the lubricant oil

will increase its viscosity and cause the shaft to be seized. Copper plating

may also result in seizing of shaft.

b) Seizing of piston

This is caused by too small fit clearance between the piston and

cylinder or expansion due to heat.

Judgement of seizing shaft and piston: After the refrigerator has been

powered on, the compressor will not start and run, but a slight hum sound

can be heard, and several seconds later, thermal protective relay will

actuate and make the contacts open; this process will occur repeatedly, but

the compressor cannot be started.

E. Compressor won't stop

Sometimes, the compressor will run continuously(for several hours or

run without end), If the food placed in the refrigerator is not too much, there

may be the following two situations: i) the refrigerator very low, this means

that the control system is probably out of order; ii) the control system works

normally, and there are troubles in the refrigerating system or other parts.

1) Temperature is set improperly

a)  The temperature control knob is set to the "coldest" position. This

26

Maintenance Service and Trouble Shooting

position is to be used for fast freezing or continuous running, its

temperature for power off is too low, therefore, the compressor won't stop

and the temperature in the refrigerator becomes lower and lower .

b)  Inspection method: check the temperature control knob to see

whether it is set in the position "coldest".

2)  Thermostat is malfunctioning and makes the compressor running

continuously

Analysis of Trouble

When the thermostat doesn't work normally, it will make the compressor

run continuously, and hence very low temperature will be achieved in the

refrigerator. The fault is generally due to the fact that the contacts of the

thermostat cannot be released.

Remedy

Dismantle the thermostat and make a through checkup, if it is totally out

of functioning, replace it with a new one.

3)  evaporation temperature is too high in refrigerating system, resulting

in lower refrigerating capacity and hence continuous running of compressor

Analysis of Trouble

Leakage of refrigerant and clogging in a refrigerating system will directly

affect its refrigerating capacity. Due to the reduction in its refrigerating

capacity, the refrigerator temperature cannot reach its rated value, the

thermostat won't work, thus the compressor runs continuously. When the

evaporation temperature in the system is too high, the temperature sensing

agent in the temperature sensor is also hotter, therefore, the thermostat is

unable to cut off the power supply to the compressor and stop it.

Remedy

If it has been found that the refrigerant in the system is insufficient in

quantity, recharge it with refrigerant. In case clogging occurs, disassemble

the part where it is blocked. If the evaporation temperature is too high,

settle this problem with an appropriate quantity of refrigerant.

4)  No stoppage of compressor due to damaged heat-insulation layer

inside case body and/or door seal

Analysis of Trouble

When the heat-insulation layer inside the case body deteriorates or the

door seal is not closely touches the door frame, the temperature in the

refrigerator will rise and makes the compressor running continuously.

Remedy

Check the heat-insulation layer for its damaged parts, repair it to

improve its heat-insulation property. If the door has deformed or the door

seal is not tightly sealed against the case body, repair them, respectively.

5)  Too much food is placed in the refrigerator, or is placed too densely,

resulting in poor ventilation or poor conditions for temperature sensing, and

hence no stoppage of the compressor.

6)  Too high ambient temperature, poorer ventilation and heat

27

Maintenance Service and Trouble Shooting

dissipation make the compressor running without stop.

F. Electric leakage of refrigerator

1)  Slight electric leakage

Electric insulation has deteriorated due to being affected with damp,

thus resulting in slight electric leakage.

2)  Serious electric leakage

Refrigerator case has become live due to faults occurred in some

electric devices or the erroneous wiring in installing power cord plug or

outlet by the user. This is very dangerous.

3)  Electric leakage test

a)  Slight electric leakage

A tingle sense will be experienced as soon as you touch the metal parts

of the refrigerator with your hand .When you test them with a test

pencil ,its neon lamp will come on .In that case ,the first thing you need to

do is to determine whether the grounding is perfect .If the grounding is no

problem ,turn off the refrigerator immediately ,then check the insulation of

electric circuits with an avometer .

b)  Serious electric leakage

Never touch the case body of the refrigerator ,its door handle or other

metal parts with your hand .Test the refrigerator with a test pencil ,it will

light up intensively ;measure the resistance between the power cord plug

and the case body with an avometer ,the reading will be zero (0); in the

worst case ,the fuse will be blown .Check the 3-prong outlet to see whether

the live wire and the null line are inversely connected ,this makes the

ground protection lead-out from the power cord plug being connected to the

live wire .Another possibility is that the live wire and null line of the outdoor

power supply circuit have been inversely connected accordingly ,this

makes the null line become a live wire .

G .Stronger vibration and loader noise

1)  Refrigerator placed improperly

a)  Uneven ground

Uneven ground will cause there frigerator to be placed

unsteadily ,causing stronger vibration and noise during its operation .

b)  Leveling screws not properly adjusted

If  the leveling screws on refrigerator legs have not been adjusted

properly ,vibration and noise will still occur even if the refrigerator has been

placed on a level ground .

2)  Abnormal noise from compressor

Three suspended spring inside the compressor case are out of

balance ,and strike against the case ,moreover ,the wear of compressor

parts may cause noise sometimes .

3)  Resonance of tubing and loosening of parts

Improper and compact laying of tubes or the loosening of parts may

cause vibration and noise .

28

Maintenance Service and Trouble Shooting

4)  Inspection method

To find the sources of noise, press the vibration spot with your hand

while the refrigerator is in operation and listen attentively whether the

vibration becomes weake or vanishes. If the refrigerator has not been

levelly placed ,put a level meter on its top table and adjust the leveling

screws on its legs .In case noise occurs from the compressor ,strike

different locations on the side surface of its case using a rubber hammer or

hand hammer with a wood block in-between so as to determine whether

the suspended springs are out of balance or being seized .

29

数量

1

1

1

2

1

1

1

1